Balancing and Settlement Code BSC PROCEDURE BSCP533 – Appendix A: PARMS Data Provider File Formats Version 17.1 Date:

BSCP533 Appendix A

Relating to

PARMS Data Provider File Formats

1.	Reference is made to the Balancing and Settlement Code and in particular, to the definition of “BSC Procedure” In Section X, Annex X-1 thereof.

2.	This is BSC Procedure 533 Appendix A Version 17.1 relating to PARMS Data Provider File Formats.

3.	This BSC Procedure is effective from.

4.	This BSC Procedure has been approved by the Panel.

Intellectual Property Rights, Copyright and Disclaimer The copyright and other intellectual property rights in this document are vested in Elexon or appear with the consent of the copyright owner. These materials are made available for you for the purposes of your participation in the electricity industry. If you have an interest in the electricity industry, you may view, download, copy, distribute, modify, transmit, publish, sell or create derivative works (in whatever format) from this document or in other cases use for personal academic or other non-commercial purposes. All copyright and other proprietary notices contained in the document must be retained on any copy you make. All other rights of the copyright owner not expressly dealt with above are reserved. No representation, warranty or guarantee is made that the information in this document is accurate or complete. While care is taken in the collection and provision of this information, Elexon Limited shall not be liable for any errors, omissions, misstatements or mistakes in any information or damages resulting from the use of this information or action taken in reliance on it.

AMENDMENT RECORD

| Version | Date | Description of Changes | Changes Included | Mods /Panel/ Committee Refs |
| --- | --- | --- | --- | --- |
| 0.1 | Code Effective Date | Re-badged AP533 to form BSCP533 |  |  |
| 1.1 | Code Effective Date | Submitted to the Panel for approval | NCR313 |  |
| 2.0 | 27/03/01 | Approved by the Panel on 22nd February 2001 |  | P13/008 |
| 3.0 | 01 November | Changes for Modification P68 | P68 | NPAB19/210 |
| 4.0 | 01/03/03 | Updated to reflect the terminology used in the BSC | CP852 | NPAB22/244 |
| 5.0 | 01/08/03 | Updated for Modification P62 | P62 |  |
| 6.0 | 06/01/04 | Updated for Modification P99 | P99 |  |
| 6.1 | 13/01/04 | Updated with SVG and P99 workshop comments | P99 |  |
| 7.0 | 01/05/04 | Approved by SVG | P99 |  |
| 8.0 | 01/07/04 | Consistency Amendments and Working Day timings updates | P99 | SVG41/002 |
| 9.0 | 04/11/04 | Incremented to align with document suite amended for the SVA November 2004 Release |  |  |
| 10.0 | 23/02/05 | SVA February 05 Release and BETTA 6.3 | BETTA 6.3, CP1090, CP1091 | SVG47/004 |
| 11.0 | 30/06/05 | SVA June 05 Release | CP1103 | SVG52/005 |
| 12.0 | 03/11/05 | SVA November 05 Release | CP1087 | SVG56/004 |
| 13.0 | 28/06/07 | June 07 Release Updated terminology in preparation for P197 Release | CP1178 P197 | SVG72/04 |
| 14.0 | 05/11/09 | November 09 Release | CP1248 v2.0 | SVG97/01 |
| 15.0 | 01/11/10 | November 10 Release | CP1325 | SVG111/01 |
| 16.0 | 01/07/11 | June 11 Release | CP1334 | SVG114/02 |
|  |  |  | CP13441 | SVG122/02 |
|  |  |  | CP13481 | SVG125/01 |
| 17.0 | 07/11/13 | November 2013 Release | CP1387 | PAB145/08 SVG145/04 |
|  |  |  |  |  |  | CP1399 |  | PAB152/04 SVG152/08 |  |
| 17.1 |  | REC Transition |  |  |

Contents

1.	INTRODUCTION	6

1.1	Purpose	6

1.2	PARMS Data	6

1.2.1	PARMS Data: General Description	6

2.	PHYSICAL FILE PRESENTATION	7

2.1	Media	7

2.2	File Naming	7

2.3	File Submission and Verification of Output Data	7

2.4	File Resubmission and Correction	8

3.	PHYSICAL FILE FORMATS	8

3.1	Pool File Format	8

4.	STANDING DATA FILE FORMATS	12

4.1	DPI - Data Provider Information	12

4.2	PARMS Market Domain Data	13

4.3	Suppliers Trading / Ceased Trading in GSP Groups	15

5.	OUTPUT DATA FILE FORMATS	16

5.1	TA01 – GSP Group Correction Factor	16

5.2	TA02 – Annual Demand Ratio	17

5.3	CM01 – CVA MOA Proving Tests	18

5.4	CM02 – CVA MOA Fault Resolution	19

5.5	SP11 – Timely Appointment of Agents	20

5.6	SP12 – Timely Notification of Changes of the Data Aggregator via D0148	21

5.7	SP13 - Timely Notification of Changes of the Meter Operator Agent via D0148	22

5.8	SP14 - Timely Notification of Changes of the Data Collector via D0148	23

5.9	SP04 – Installation of HH Metering	24

5.10	SP15 - Missing Appointments of Agents	25

5.11	Not Used	25

5.12	SP07 – SMRA & SVAA MSID Count – SMRA File	26

5.13	SP07 – SMRA & SVAA MSID Count – SVAA File	27

5.14	SP08 – Energy and MSIDs on Actuals	28

5.15	SP09 – NHH Defaults	30

5.16	HM11 - Timely Sending of HH MTDs to HHDCs	31

5.17	HM12 - Missing HH MTDs	32

5.18	HM13 – Quality of HH MTDs	33

5.19	HM14 - Timely HH Meter Investigation Requests	34

5.20	NM11 - Timely Sending of NHH MTDs to NHHDCs	35

5.21	NM12 – Missing NHH MTDs	36

5.22	NC11 - Missing NHH Meter Reads & History from Old NHHDC to New NHHDC	37

Appendix 1:	Algorithm For Checksum Required For BSCCo Defined Software	39

Related Documents

| Reference 1 | PARMS User Requirements Specification |
| --- | --- |
| Reference 2 | BSC Procedure: PARMS Data Provision, Reporting and Publication of Peer Comparison Data (BSCP533) |
| Reference 3 | BSC Procedure: PARMS Data Provision (BSCP533 – Appendix B: PARMS Calculation Guidelines) |

# 1.	INTRODUCTION

## 1.1	Purpose

The purpose of this document is to specify the file format specification associated with the information to be submitted to the Performance Assurance Reporting and Monitoring System (PARMS) which monitors Market Participants’ performance. This is intended to provide guidance for Data Providers to assist them in the development of their systems.

## 1.2	PARMS Data

### [MEM]1.2.1	PARMS Data: General Description

PARMS Data consists of data pertaining to the performance of specified market Participants and is provided via a pre-determined series of files by agreed Data Providers (SMRAs, the SVAA, the CDCA, Suppliers or Supplier Agents). This data is specified in BSCP533 PARMS Data Provision, Reporting and Publication of Peer Comparison Data. The data will be loaded automatically into the PARMS database using the corresponding PARMS validation process.

The data descriptions defined in the relevant Data sections of BSCP533 have been summarised in this paper into usable data identifiers.

The appropriate files are summarised below:

| Output Data |  |  |
| --- | --- | --- |
| Serial | Titled | FILETYPE |
| TA01 | GSP Group Correction Factor | P0137001 |
| TA02 | Annual Demand Ratio | P0138001 |
| CM01 | CVA MOA Proving Tests | P0133001 |
| CM02 | CVA MOA Fault Resolution | P0134001 |
| SP01 | Delivery of Routine Performance Reports | P0139001 |
| SP02 | Delivery of Routine Performance Logs | P0140001 |
| SP04 | Installation of HH Metering | P0142001 |
| SP07 | SMRA & SVAA MSID Count – SMRA File | P0045002 (SMRA) |
| SP07 | SMRA & SVAA MSID Count – SVAA File | P0164001 (SVAA) |
| SP08 | Energy and MSIDs on Actuals | P0145002 |
| SP09 | NHH Defaults | P0146001 |
| SP11 | Timely Appointment of Agents | P0224001 |
| SP12 | Timely Notification of Changes of the Data Aggregator via D0148 | P0225001 |
| SP13 | Timely Notification of Changes of the SVA Meter Operator Agent via D0148 | P0226001 |
| SP14 | Timely Notification of Changes of the Data Collector via D0148 | P0227001 |
| SP15 | Missing Appointments of Agents | P0228001 |
| HM11 | Timely Sending of HH MTDs to HHDCs | P0229001 |
| HM12 | Missing HH MTDs | P0230001 |
| HM13 | Quality of HH MTDs | P0231001 |
| HM14 | Timely HH Meter Investigation Requests | P0232001 |
| NM11 | Timely Sending of NHH MTDs to NHHDCs | P0233001 |
| NM12 | Missing NHH MTDs | P0234001 |
| NC11 | Missing NHH Meter Reads & History from Old NHHDC to New NHHDC | P0235001 |
| Standing Data |  | FILETYPE |
| DPI - Data Provider Information |  | P0135001 |
| PARMS Market Domain Data |  | P0136001 |
| Suppliers Trading / Ceased Trading in GSP Groups |  | P0127001 |

# 2.	PHYSICAL FILE PRESENTATION

## 2.1	Media

Data files will be submitted into PARMS by the Data Provider (Supplier, Supplier Agent or BSC Agent) in the form of ASCII files. A separate data file is required for each Serial, although a number of files may be contained in one submission.

## 2.2	File Naming

An 8-character file naming convention will be used as follows:

- The 1st to 4th characters will be the participant ID of the Data Provider;

- The 5th to 7th will relate to the File Identifier that will be used in the SVA Data Catalogue; and

- The 8th will be the last digit of the year number (e.g. ‘2’ for 2002, ‘3’ for 2003).

The file extension will indicate the month to which the data pertains (JAN, FEB, MAR, APR, MAY, JUN, JUL, AUG, SEP, OCT, NOV, DEC).

## 2.3	File Submission and Verification of Output Data

Data Providers should submit PARMS data +(x) WDs after the end of the reporting period as follows:

- Suppliers and Supplier Agents Routine Performance Monitoring Report: +20 WDs;

- SMRAs: +10 WDs; and

- SVAA: +7 WDs

All files will be submitted direct to PARMS by the Data Providers specified in the File Formats. Where there has been no occurrence of an event monitored by a Serial, Data Providers should reflect this by using zeros in place of the values for each Supplier and GSP Group. Where there has been no occurrence of an event, but appointments exist in for that Supplier and GSP Group combination, Data Providers should reflect this by submitting zero values against all the standards for these combinations.  Failure to do so may result in missing data.  If no data is provided at all then PARMS will regard the submission as incomplete and apply Supplier Charges if appropriate. The process and timescale for submission is detailed in BSCP533 ‘PARMS Data Provision Reporting and Publication of Peer Comparison Data’.

Once received, BSCCo will distribute any Supplier-related data to the relevant Suppliers in order to allow them to verify the data submitted by its appointed agents, particularly where poor performance against a Serial may lead to Supplier Charges. Any queries raised will be dealt with between the Supplier and its Agent, and any resubmission of data should be by the specified Data Provider. (Suppliers can, of course, request that their agents provide copies of any files submitted to PARMS for checking, but this will not be assumed by BSCCo).

## 2.4	File Resubmission and Correction

Once a Data Provider has submitted a file, it may be resubmitted in order to correct errors subsequently identified in the file. For each Serial, if a correction is required then a complete submission for the affected GSP Group must be provided, such that it is made clear that the data that was originally correct should remain in the system. If a file is submitted containing only the corrected data, PARMS will assume that the rest of the data already stored in the system has been since been identified by the Data Provider as incorrect and that the data should be deleted from the system.

# 3.	PHYSICAL FILE FORMATS

## 3.1	Pool File Format

The majority of the Data Output files will use the Pool file format. In this format, a file contains a number of records, each starting with a three-character identifier and ending with a Record Delimiter character. The first record of each file will be a header; the last a footer. The last record of a physical block will not require a Record Delimiter.

Each record contains fields of various types such as text, integer, date and time. The full range is described below:

| Type | Format | Example |
| --- | --- | --- |
| int (Integer) | ASCII representation, no leading zeros or spaces, leading “-“ if negative (no sign if positive) | -1234 12 |
| dec (Decimal) | ASCII representation. As for Integer, but with a decimal point and fixed number of decimal digits (including trailing zeros) dependent on precision | dec(4,2): -12.34 dec(3,2): 1.20 |
| text | ASCII format, left aligned with trailing spaces stripped. Only includes printable characters excluding the separator | The quick fox |
| date | ASCII format as: YYYYMMDD | 19961216 |
| time | ASCII format as: HHMMSS (24 hour format). Note: both GMT and local time (e.g. British Summertime) will be used and will be indicated as necessary. | 131501 |
| date/time | ASCII format as: YYYYMMDDHHMMSS | 19961216131501 |
| bol (Boolean) | One ASCII character: T for True, F for False (uppercase only) | T F |

A Field Separator character separates each field (with the exception of the last field of each record). All fields are mandatory, unless specifically indicated as optional in the ‘comments’ column. A null submission is achieved by omitting any characters between the Field Separators.

All files sent or received by PARMS are structured as follows:

- Header - first record in file - record type = “ZHD”

- Body - other file records

- Footer - last record in file - record type = “ZPT”

For PARMS Output Data, the first record of the file “Body” is a subject participant header (record type “SUB”) containing information about the subject market participant.

Note that there may be many SUB records (e.g. where a Supplier has a number of agents, or an agent contracted to many Suppliers, and where the Supplier is operating in more than one GSP Group).

The components of these three standard record types are defined in the following tables:

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | 5 character type (ranges allocated for DTS, pool or internal use) plus 3 character version |
| 3 | From Role Code | text(1) |  |
| 4 | From Participant Id | text(4) |  |
| 5 | To Role Code | text(1) | ‘Z’ (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) | ‘POOL’ |
| 7 | Creation Time | date/time | Time file processing was started. Specified in GMT. |

| SUB - Subject Participant Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = H (half hourly) = N (non-half hourly) |
| 3 | Market Participant Role Code | text(1) | Role Code of the subject Market Participant |
| 4 | Market Participant Id | text(4) | Identifier of the subject Market Participant |
| 5 | Period End Date | date | Date of the last calendar day of the period (generally either a month or a quarter) to which the data applies. |
| 6 | Periodicity | text(1) | Indicates whether the Period End Date is ‘W’eekly, ‘M’onthly or ‘Q’uarterly. |

| ZPT - File Footer |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZPT |
| 2 | Record count | int(10) | Includes header and footer |
| 3 | Checksum | int(10) | Although type is shown as int(10) the value is actually a 32-bit unsigned value and hence will fit in an “unsigned long” C variable. |

The remaining component of the File is a body record containing the required PARMS information. These will be specific for each Serial and, like the SUB record, will be repeated in situations where, for example, an SMRS or a Supplier Agent is operating in a number of different GSP Groups, or where data is required for a number of different Settlement Run types.

Some files involve the reporting of a data item against a list of Settlement dates, such as for Serial SP07. In these instances, the dates and the associated data items should be listed in ascending order.

The character set used is based on the ISO Level B character set and will include the following characters:

| Letters, upper case | A to Z |
| --- | --- |
| Letters, lower case | a to z |
| Numerals | 0 to 9 |
| Space character |  |
| Full stop | . |
| Comma | , |
| Hyphen/minus | - |
| Opening parenthesis | ( |
| Closing parenthesis | ) |
| Slash | / |
| Apostrophe | ' |
| Plus | + |
| Colon | : |
| Equals | = |
| Question mark | ? |
| Exclamation mark | ! |
| Quotation mark | " |
| Percentage sign | % |
| Ampersand | & |
| Asterisk | * |
| Semi-colon | ; |
| Less than | < |
| Greater than | > |
| Underscore | _ |

Field Separator: The vertical bar character ‘|’ will be used as the separator.

Record Delimiter: The Line Feed character (hex “A”) or a Carriage Return is used as the delimiter.

# 4.	STANDING DATA FILE FORMATS

## 4.1	DPI - Data Provider Information

This file provides PARMS with the identities of Supplier Agents that will be providing Output Data for that Supplier in each relevant GSP Group.

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type |  | Comments |
| 1 | Record Type | text(3) |  | = ZHD |
| 2 | File Type | text(8) |  | = P0135001 |
| 3 | From Role Code | text(1) |  | = X (Supplier) |
| 4 | From Participant Id | text(4) |  | = ID of originating Supplier |
| 5 | To Role Code | text(1) |  | = Z (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) |  | = POOL |
| 7 | Creation Time | date/time |  | Date & time of file generation |
| DPI - Data Provider Information |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = DPI |  |
| 2 | GSP Group | text(2) |  |  |
| 3 | Serial reference | text(4) | e.g. HM11, NM11, SP04, etc |  |
| 4 | Data Provider ID | text(4) |  |  |
| 5 | Data Provider Role Code | text(1) |  |  |
| 6 | Period end date | date | = Calendar date of end of reporting period |  |
| ZPT – File Footer |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZPT |  |
| 2 | Record count | int(10) | Includes header and footer |  |
| 3 | Checksum | int(10) | Although type is shown as integer(10) the value is actually a 32-bit unsigned value and hence will fit in an “unsigned long” C variable. |  |

Backus-Naur Form:

Data Provider Information ::= ZHD {DPI} ZPT

## 4.2	PARMS Market Domain Data

| ZHD - File Header |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Field | Field Name | Type |  | Comments |  |
| 1 | Record Type | text(3) |  | = ZHD |  |
| 2 | File Type | text(8) |  | = P0136001 |  |
| 3 | From Role Code | text(1) |  | = G (SVA Agent) |  |
| 4 | From Participant Id | text(4) |  | CAPG |  |
| 5 | To Role Code | text(1) |  | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) |  | = POOL |  |
| 7 | Creation Time | date/time |  | Date & time of file generation |  |
| VER : MDD version |  |  |  |  |  |
| 1 | Record Type | text(3) |  | = VER |  |
| 2 | MDD Version Number | int(8) |  |  |  |
| GSG : GSP Group |  |  |  |  |  |
| 1 | Record Type | text(3) |  | = GSG |  |
| 2 | GSP Group Id | text(2) |  |  |  |
| 3 | GSP Group Name | text(30) |  |  |  |
| GGD : GSP Group Distributors |  |  |  |  |  |
| 1 | Record Type | text(3) |  | =GGD |  |
| 2 | Distributor Id | int(2) |  |  |  |
| 3 | Market Participant Role Code | text(1) |  |  |  |
| 4 | Effective from Date {MPR} | date |  |  |  |
| 5 | Effective from Settlement Date {GGD} | date |  |  |  |
| 6 | Effective to Settlement Date {GGD} | date |  | optional |  |
| MRC : Market Participant Role Codes |  |  |  |  |  |
| 1 | Record Type | text(3) |  | = MRC |  |
| 2 | Market Participant Role Code | text(1) |  |  |  |
| 3 | Role Code Description | text(30) |  |  |  |
| MAP : Market Participants |  |  |  |  |  |
| 1 | Record Type | text(3) |  | = MAP |  |
| 2 | Market Participant Id | text(4) |  |  |  |
| 3 | Market Participant Name | text(40) |  |  |  |
| 4 | Pool Member Id | text(4) |  | optional |  |
| MPR : Market Participant Roles |  |  |  |  |  |
| 1 | Record Type |  | text(3) |  | = MPR |
| 2 | Market Participant Role Code |  | text(1) |  |  |
| 3 | Effective from Settlement Date {MPR} |  | date |  |  |
| 4 | Effective to Settlement Date {MPR) |  | date |  | optional |
| SSR : SSR Run Type |  |  |  |  |  |
| 1 | Record Type |  | text(3) |  | = SSR |
| 2 | SSR Run Type |  | text(2) |  |  |
| 3 | SSR Run Type Name |  | text(40) |  |  |
| SSC : Settlement Calendar |  |  |  |  |  |
| 1 | Record Type |  | text(3) |  | = SSC |
| 2 | SSR Run number |  | int(7) |  |  |
| 3 | Settlement Date |  | date |  |  |
| 4 | SSR Run Type |  | text(2) |  |  |
| 5 | SSR Run Date |  | date |  |  |

Note

1	there is a one to many relationship between Meter asset provider (MAP) and market participant role (MPR) record types

2	the VER record type denotes which version of MDD was used for the source of this file

Backus-Naur Form:

PARMS Market Domain Data ::= ZHD VER {GSG {GGD}} {MRC} {MAP {MPR}} {SSR} {SSC} ZPT

## 4.3	Suppliers Trading / Ceased Trading in GSP Groups

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0127001 |
| 3 | From Role Code | text(1) | = G (SVA Agent) |
| 4 | From Participant Id | text(4) | CAPG |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SPT : Supplier Trading / Ceased Trading in GSP Group |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SPT |
| 2 | GSP Group Id | text(2) |  |
| 3 | Supplier Id | text(4) |  |
| 4 | Effective from Date Supplier Trading in GSP Group | date | Calendar date started trading in GSP Group |
| 5 | Effective to Date Supplier Trading in GSP Group | date | Optional. Calendar date ceased trading in GSP Group |

Backus-Naur Form:

Suppliers Trading / Ceased Trading in GSP Groups ::= ZHD {SPT} ZPT

# 5.	OUTPUT DATA FILE FORMATS

## 5.1	TA01 – GSP Group Correction Factor

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0137001 |
| 3 | From Role Code | text(1) | = G (SVAA) |
| 4 | From Participant Id | text(4) | = CAPG |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = B (indicates HH and NHH data) |
| 3 | Market Participant Role Code | text(1) | = NULL |
| 4 | Market Participant Id | text(4) | = NULL |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| TA1 Trading Arrangements Serial 1 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = TA1 |
| 2 | Number of GCF queries raised | int(5) |  |

## 5.2	TA02 – Annual Demand Ratio

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0138001 |
| 3 | From Role Code | text(1) | = G (SVAA) |
| 4 | From Participant Id | text(4) | =CAPG |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = B (indicates HH and NHH data) |
| 3 | Market Participant Role Code | text(1) | Not applicable |
| 4 | Market Participant Id | text(4) | Not applicable |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| TA2 Trading Arrangements Serial 2 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = TA2 |
| 2 | Annual Demand Ratio | dec(5,4) |  |

Backus-Naur Form:

Annual Demand Ratio ::= ZHD SUB TA2 ZPT

## 5.3	CM01 – CVA MOA Proving Tests

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0133001 |
| 3 | From Role Code | text(1) | = Z |
| 4 | From Participant Id | text(4) | = CDCA |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SB1 |
| 2 | Market Sector | text(1) | = H |
| 3 | Market Participant Role Code | text(1) | = M (MOA) |
| 4 | Market Participant Id | text(8) | ID of CVA MOA |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| CM01 CVA MOA Serial 1 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = CM1 |
| 2 | GSP Group Id | text(2) | = ‘NULL’ if directly-connected site |
| 3 | Number of MSIDs affected in period | int(7) |  |
| 4 | Average number of working days Proving Test is outstanding after Effective From Date at time of report | dec(4,1) |  |
| 5 | Count of faults outstanding after Effective From Date | int(7) |  |

Backus-Naur Form:

CVA MOA Proving Tests ::= ZHD {SB1 {CM1}} ZPT

## 5.4	CM02 – CVA MOA Fault Resolution

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0134001 |
| 3 | From Role Code | text(1) | = Z |
| 4 | From Participant Id | text(4) | = CDCA |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SB2 |
| 2 | Market Sector | text(1) | = H |
| 3 | Market Participant Role Code | text(1) | = M (MOA) |
| 4 | Market Participant Id | text(8) | ID of CVA MOA |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| CM02 CVA MOA Serial 2 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = CM2 |
| 2 | GSP Group Id | text(2) | = ‘NULL’ if directly-connected site |
| 3 | Number of MSIDs affected in period | int(7) |  |
| 4 | Count of faults identified | int(7) |  |
| 5 | Average number of working days faults outstanding at time of report | dec(4,1) |  |
| 5 | Average number of working days taken to resolve faults | dec(4,1) |  |

Backus-Naur Form:

CVA MOA Fault Resolution ::= ZHD {SB2 {CM2}} ZPT

## 5.5	SP11 – Timely Appointment of Agents

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0224001 |  |
| 3 | From Role Code | text(1) | = C, D, M |  |
| 4 | From Participant Id | text(4) | ID of Supplier Agent |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = N (NHH data) or H (HH data) |  |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |  |
| 4 | Market Participant Id | text(4) | ID of Supplier |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| SP11 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = X11 |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Count of D0155s received within Reporting Period | int(7) |  |  |
| 4 | Count of D0155s not received before Agent EFD | int(7) |  |  |
| 5 | Count of D0155s received before SF | int(7) |  |  |
| 6 | Count of D0155s received before R1 | int(7) |  |  |
| 7 | Count of D0155s received before R2 | int(7) |  |  |
| 8 | Count of D0155s received before R3 | int(7) |  |  |
| 9 | Count of D0155s received before RF | int(7) |  |  |
| 10 | Count of D0155s received after RF | int(7) |  |  |

Backus-Naur Form:

Timely Appointment of Agents::= ZHD {SUB {X11}} ZPT

## 5.6	SP12 – Timely Notification of Changes of the Data Aggregator via D0148

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0225001 |  |
| 3 | From Role Code | text(1) | = C or D |  |
| 4 | From Participant Id | text(4) | ID of Supplier Agent |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = N (NHH data) or H (HH data) |  |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |  |
| 4 | Market Participant Id | text(4) | ID of Supplier |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| SP12 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = X12 |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Count of D0148s received within Reporting Period | int(7) |  |  |
| 4 | Count of D0148s not received before DA EFD | int(7) |  |  |
| 5 | Count of D0148s received before SF | int(7) |  |  |
| 6 | Count of D0148s received before R1 | int(7) |  |  |
| 7 | Count of D0148s received before R2 | int(7) |  |  |
| 8 | Count of D0148s received before R3 | int(7) |  |  |
| 9 | Count of D0148s received before RF | int(7) |  |  |
| 10 | Count of D0148s received after RF | int(7) |  |  |

Backus-Naur Form:

Timely Notification of Changes of the DAs via D0148::= ZHD {SUB {X12}} ZPT

## [MEM]5.7	SP13 - Timely Notification of Changes of the SVA Meter Operator Agent via D0148

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0226001 |  |
| 3 | From Role Code | text(1) | = C or D |  |
| 4 | From Participant Id | text(4) | ID of Supplier Agent |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = N (NHH data) or H (HH data) |  |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |  |
| 4 | Market Participant Id | text(4) | ID of Supplier |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| SP13 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = X13 |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Count of D0148s received within Reporting Period | int(7) |  |  |
| 4 | Count of D0148s not received before SVA MOA EFD | int(7) |  |  |
| 5 | Count of D0148s received before SF | int(7) |  |  |
| 6 | Count of D0148s received before R1 | int(7) |  |  |
| 7 | Count of D0148s received before R2 | int(7) |  |  |
| 8 | Count of D0148s received before R3 | int(7) |  |  |
| 9 | Count of D0148s received before RF | int(7) |  |  |
| 10 | Count of D0148s received after RF | int(7) |  |  |

Backus-Naur Form:

Timely Notification of changes of the SVA MOAs via D0148::= ZHD {SUB {X13}} ZPT

## 5.8	SP14 - Timely Notification of Changes of the Data Collector via D0148

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0227001 |  |
| 3 | From Role Code | text(1) | = M |  |
| 4 | From Participant Id | text(4) | ID of Supplier Agent |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = N (NHH data) or H (HH data) |  |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |  |
| 4 | Market Participant Id | text(4) | ID of Supplier |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| SP14 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = X14 |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Count of D0148s received within Reporting Period | int(7) |  |  |
| 4 | Count of D0148s not received before DC EFD | int(7) |  |  |
| 5 | Count of D0148s received before SF | int(7) |  |  |
| 6 | Count of D0148s received before R1 | int(7) |  |  |
| 7 | Count of D0148s received before R2 | int(7) |  |  |
| 8 | Count of D0148s received before R3 | int(7) |  |  |
| 9 | Count of D0148s received before RF | int(7) |  |  |
| 10 | Count of D0148s received after RF | int(7) |  |  |

Backus-Naur Form:

Timely Notification of changes of the DCs via D0148::= ZHD {SUB {X14}} ZPT

## 5.9	SP04 – Installation of HH Metering

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0142001 |
| 3 | From Role Code | text(1) | = X (Supplier) |
| 4 | From Participant Id | text(4) | ID of Supplier originating file |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = H (indicates HH data) |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |
| 4 | Market Participant Id | text(4) | ID of Supplier |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| SP4 Supplier Serial 4 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SP4 |
| 2 | GSP Group Id | text(2) |  |
| 3 | MSID | int(13) |  |
| 4 | Aggregated Standard Installed MSID-Days in Month HH 100kW Site | int(4) |  |
| 5 | Aggregated Not Installed MSID-Days in Month HH 100kW Site | int(4) | Cannot exceed field 4 |
| 6 | Percentage Not Installed MSID-Days in Month HH 100kW Site | dec(4,1) |  |

Backus-Naur Form:

Installation of HH Metering::= ZHD {SUB {SP4}} ZPT

## 5.10	SP15 - Missing Appointments of Agents

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0228001 |
| 3 | From Role Code | text(1) | = C, D or M |
| 4 | From Participant Id | text(4) | ID of Supplier Agent |
| 5 | To Role Code | text(1) | = Z (Non-Core -PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = N (NHH data) or H (HH data) |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |
| 4 | Market Participant Id | text(4) | ID of Supplier |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| SP15 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = X15 |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |
| 3 | Count of D0155s held in the previous 14 months | int(7) |  |
| 4 | Count of D0155s for which no D0148s are held | int(7) |  |
| 5 | Count of D0148s missing before R1 | int(7) |  |
| 6 | Count of D0148s missing before R2 | int(7) |  |
| 7 | Count of D0148s missing before R3 | int(7) |  |
| 8 | Count of D0148s missing before RF | int(7) |  |
| 9 | Count of D0148s missing after RF | int(7) |  |

Backus-Naur Form:

Missing Appointments of Agents::= ZHD {SUB SP5X15} ZPT

## 5.11	Not Used

## 5.12	SP07 – SMRA & SVAA MSID Count – SMRA File

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0045002 |
| 3 | From Role Code | text(1) | = P (SMRA) |
| 4 | From Participant Id | text(4) | ID of originating SMRA |
| 5 | To Role Code | text(1) | = Z (Non-Core -PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = B (indicates HH and NHH data) |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |
| 4 | Market Participant Id | text(4) | ID of Supplier |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| SP7 Supplier Serial 7 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SP7 |
| 2 | GSP Group Id | text(2) |  |
| 3 | Market Participant ID | text(4) | ID of Data Aggregator |
| 4 | Market Participant Role Code | text(1) | A (Half Hourly Data Aggregator) or B (non-Half Hourly Data Aggregator) |
| 5 | Date | date(8) | yyyymmdd |
| 6 | Energised MSID Count | int (10) |  |
| 7 | De-energised MSID Count | int(10) |  |

Backus-Naur Form:

SMRA/SVAA MSID Count– SMRA File::= ZHD {SUB {SP7}} ZPT

## 5.13	SP07 – SMRA & SVAA MSID Count – SVAA File

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0164001 |
| 3 | From Role Code | text(1) | = G (SVAA) |
| 4 | From Participant Id | text(4) | = CAPG |
| 5 | To Role Code | text(1) | = Z (Non-Core -PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = B (indicates HH and NHH data) |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |
| 4 | Market Participant Id | text(4) | ID of Supplier |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| SP7 Supplier Serial 7 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SP7 |
| 2 | GSP Group Id | text(2) |  |
| 3 | Market Participant ID | text(4) | ID of Data Aggregator |
| 4 | Market Participant Role Code | text(1) | A (Half Hourly Data Aggregator) or B (non-Half Hourly Data Aggregator) |
| 5 | Settlement Date | date(8) | yyyymmdd |
| 6 | Settlement Type | text(2) |  |
| 7 | MSID Count | int(10) |  |

Backus-Naur Form:

SMRA/SVAA MSID Count– SVAA File::= ZHD {SUB {SP7}} ZPT

## 5.14	SP08 – Energy and MSIDs on Actuals

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0145002 |
| 3 | From Role Code | text(1) | = G (SVAA) |
| 4 | From Participant Id | text(4) | = CAPG |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = B (indicates HH and NHH data) |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |
| 4 | Market Participant Id | text(4) | ID of Supplier |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| SP8 Supplier Serial 8 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SP8 |
| 2 | Settlement Day | date |  |
| 3 | Settlement Type | text(2) | SF, R1, R2, R3 and RF run types |
| 4 | GSP Group Id | text(2) |  |
| 5 | % NHH Energy Aggregated on Actuals | dec(4,1) | The type ‘dec 4,1’ allows for percentage values up to, and including, 999.9% in this data field |
| 6 | % NHH MSIDs Aggregated on Actuals | dec(4,1) |  |
| 7 | Total Actual NHH Energy | dec(10,2) |  |
| 8 | Total NHH Energy | dec(10,2) |  |
| 9 | % non-100kW HH Energy Aggregated on Actuals | dec(4,1) | The type ‘dec 4,1’ allows for percentage values up to, and including, 999.9% in this data field |
| 10 | % non-100kW HH MSIDs Aggregated on Actuals | dec(4,1) |  |
| 11 | Total Actual non-100kW HH Energy | dec(10,2) |  |
| 12 | Total non-100kW HH Energy | dec(10,2) |  |
| 13 | % 100kW HH Energy Aggregated on Actuals | dec(4,1) | The type ‘dec 4,1’ allows for percentage values up to, and including, 999.9% in this data field |
| 14 | % 100kW HH MSIDs Aggregated on Actuals | dec(4,1) |  |
| 15 | Total Actual 100kW HH Energy | dec(10,2) |  |
| 16 | Total 100kW HH Energy | dec(10,2) |  |

Backus-Naur Form:

Energy and MSIDs on Actuals::= ZHD {SUB {SP8}} ZPT

## 5.15	SP09 – NHH Defaults

| ZHD - File Header |  |  |  |
| --- | --- | --- | --- |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = ZHD |
| 2 | File Type | text(8) | = P0146001 |
| 3 | From Role Code | text(1) | = G (SVAA) |
| 4 | From Participant Id | text(4) | = CAPG |
| 5 | To Role Code | text(1) | = Z (Non-Core -PARMS) |
| 6 | To Participant Id | text(4) | = POOL |
| 7 | Creation Time | date/time | Date & time of file generation |
| SUB - Subject Participant Header |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SUB |
| 2 | Market Sector | text(1) | = N (indicates NHH data) |
| 3 | Market Participant Role Code | text(1) | = X (Supplier) |
| 4 | Market Participant Id | text(4) | ID of Supplier |
| 5 | Period End Date | date | Date of last day of calendar month |
| 6 | Periodicity | text(1) | ‘M’onthly |
| SP9 Supplier Serial 9 Data |  |  |  |
| Field | Field Name | Type | Comments |
| 1 | Record Type | text(3) | = SP9 |
| 2 | Settlement Day | date |  |
| 3 | Settlement Type | text(2) | SF, R1, R2, R3 and RF run types |
| 4 | GSP Group Id | text(2) |  |
| 5 | % NHH MSIDs Settled using Default EACs | dec(4,1) |  |
| 6 | Number of NHH MSIDs Settled using Default EACs | int(7) |  |

Backus-Naur Form:

NHH Defaults ::= ZHD {SUB {SP9}} ZPT

## [MEM]5.16	HM11 - Timely Sending of HH MTDs to HHDCs

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0229001 |  |
| 3 | From Role Code | text(1) | = C |  |
| 4 | From Participant Id | text(4) | ID of Supplier Agent |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = H (indicates HH data) |  |
| 3 | Market Participant Role Code | text(1) | = M |  |
| 4 | Market Participant Id | text(4) | ID of HH SVA MOA |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| HM11 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = 1HM |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Supplier Id | text(4) | The Market Participant ID of the associated Supplier |  |
| 4 | Count of D0268s received | int(7) |  |  |
| 5 | Count of D0268s received, following a material change to the Metering System, after the MSMTD EFD before SF | int(7) |  |  |
| 6 | Count of D0268s received, following a material change to the Metering System, before R1 | int(7) |  |  |
| 7 | Count of D0268s received, following a material change to the Metering System, before R2 | int(7) |  |  |
| 8 | Count of D0268s received, following a material change to the Metering System, before R3 | int(7) |  |  |
| 9 | Count of D0268s received, following a material change to the Metering System, before RF | int(7) |  |  |
| 10 | Count of D0268s received, following a material change to the Metering System, after RF | int(7) |  |  |

Backus-Naur Form:

Timely Sending of HH MTDs to HHDCs::= ZHD {SUB {1HM}} ZPT

## [MEM]5.17	HM12 - Missing HH MTDs

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0230001 |  |
| 3 | From Role Code | text(1) | = C, M |  |
| 4 | From Participant Id | text(4) | ID of Supplier Agent (Current HHDC reporting on new HH SVA MOA; new HHDC reporting on current HH SVA MOA; new HH SVA MOA reporting on current HH SVA MOA) |  |
| 5 | To Role Code | text(1) | = Z (Non-Core -PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = H (indicates HH data) |  |
| 3 | Market Participant Role Code | text(1) | = M |  |
| 4 | Market Participant Id | text(4) | Where unknown enter ‘UUUU’ (New HH SVA MOA if reported by current HHDC; current HH SVA MOA if reported by new HHDC or new HH SVA MOA) |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| HM12 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = 2HM |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Supplier Id | text(4) | The Market Participant ID of the associated Supplier |  |
| 4 | Count of unique registrations held in previous 14 months | int(7) |  |  |
| 5 | Count of registrations where no D0268 held | int(7) |  |  |
| 6 | Count of D0268s missing before R1 | int(7) |  |  |
| 7 | Count of D0268s missing before R2 | int(7) |  |  |
| 8 | Count of D0268s missing before R3 | int(7) |  |  |
| 9 | Count of D0268s missing before RF | int(7) |  |  |
| 10 | Count of D0268s missing after RF | int(7) |  |  |

Backus-Naur Form:

Missing HH MTDs ::= ZHD {SUB {2HM}} ZPT

## [MEM[5.18	HM13 – Quality of HH MTDs

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0231001 |  |
| 3 | From Role Code | text(1) | = C |  |
| 4 | From Participant Id | text(4) | ID of HHDC |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = H (indicates HH data) |  |
| 3 | Market Participant Role Code | text(1) | = M |  |
| 4 | Market Participant Id | text(4) | ID of HH SVA MOA |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| HM13 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = 3HM |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Supplier Id | text(4) | The Market Participant ID of the associated Supplier |  |
| 3 | Count of D0268s received within Reporting Period | int(7) |  |  |
| 4 | Count of D0268s with same EFD but with a key data field change | int(7) |  |  |
| 5 | Count of affected re-sent MSIDs per MSMTD | int(7) |  |  |

Backus-Naur Form:

Quality of HH MTDs ::= ZHD {SUB {3HM}} ZPT

## [MEM[5.19	HM14 - Timely HH Meter Investigation Requests

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0232001 |  |
| 3 | From Role Code | text(1) | = C |  |
| 4 | From Participant Id | text(4) | ID of HHDC |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = H (indicates HH data) |  |
| 3 | Market Participant Role Code | text(1) | = M |  |
| 4 | Market Participant Id | text(4) | ID of HH SVA MOA |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| HM14 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = 4HM |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Supplier Id | text(4) | The Market Participant ID of the associated Supplier |  |
| 3 | Count of D0002s received in Reporting Period | int(7) |  |  |
| 4 | Count of D0002s received before SF | int(7) |  |  |
| 5 | Count of D0002s received before R1 | int(7) |  |  |
| 6 | Count of D0002s received before R2 | int(7) |  |  |
| 7 | Count of D0002s received before R3 | int(7) |  |  |
| 8 | Count of D0002s received before RF | int(7) |  |  |
| 9 | Count of D0002s received after RF | int(7) |  |  |

Backus-Naur Form:

Timely HH Meter Investigation Requests::= ZHD {SUB {4HM}} ZPT

## [MEM]5.20	NM11 - Timely Sending of NHH MTDs to NHHDCs

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0233001 |  |
| 3 | From Role Code | text(1) | = D (NHHDC) |  |
| 4 | From Participant Id | text(4) | = ID of NHHDC |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = N (indicates NHH data) |  |
| 3 | Market Participant Role Code | text(1) | = M |  |
| 4 | Market Participant Id | text(4) | ID of NHH SVA MOA |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| NM11 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = 1NM |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Supplier ID | text(4) | The Market Participant ID of the associated Supplier |  |
| 4 | Count of D0150s received within the Reporting Period | int(7) |  |  |
| 5 | Count of D0150s received, following a material change to the Metering System, before SF | int(7) |  |  |
| 6 | Count of D0150s received, following a material change to the Metering System, before R1 | int(7) |  |  |
| 7 | Count of D0150s received, following a material change to the Metering System, before R2 | int(7) |  |  |
| 8 | Count of D0150s received, following a material change to the Metering System, before R3 | int(7) |  |  |
| 9 | Count of D0150s received, following a material change to the Metering System, before RF | int(7) |  |  |
| 10 | Count of D0150s received, following a material change to the Metering System, after RF | int(7) |  |  |

Backus-Naur Form:

Timely Sending of NHH MTDs to NHHDCs::= ZHD {SUB {1NM}} ZPT

## [MEM]5.21	NM12 – Missing NHH MTDs

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0234001 |  |
| 3 | From Role Code | text(1) | = D or M (NHHDC or NHH SVA MOA) |  |
| 4 | From Participant Id | text(4) | = ID of Supplier Agent(Current NHHDC if reporting on new NHH SVA MOA; new NHHDC if reporting on current NHH SVA MOA; new NHH SVA MOA if reporting on current NHH SVA MOA) |  |
| 5 | To Role Code | text(1) | = Z (Non-Core – PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = N (indicates NHH data) |  |
| 3 | Market Participant Role Code | text(1) | =M |  |
| 4 | Market Participant Id | text(4) | ID of NHH SVA MOA, where unknown enter ‘UUUU’ (New NHH SVA MOA if reported by current NHHDC; current NHH SVA MOA if reported by new NHHDC or by new NHH SVA MOA) |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| NM12 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = 2NM |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Supplier ID | text(4) | The Market Participant ID of the associated Supplier |  |
| 4 | Count of unique registrations held in previous 14 months | int(7) |  |  |
| 5 | Count of registrations where no D0150 held | int(7) |  |  |
| 6 | Count of D0150s missing before R1 | int(7) |  |  |
| 7 | Count of D0150s missing before R2 | int(7) |  |  |
| 8 | Count of D0150s missing before R3 | int(7) |  |  |
| 9 | Count of D0150s missing before RF | int(7) |  |  |
| 10 | Count of D0150s missing after RF | int(7) |  |  |

Backus-Naur Form:

Missing NHH MTDs ::= ZHD {SUB {2NM}} ZPT

## 5.22	NC11 - Missing NHH Meter Reads & History from Old NHHDC to New NHHDC

| ZHD - File Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = ZHD |  |
| 2 | File Type | text(8) | = P0235001 |  |
| 3 | From Role Code | text(1) | = D (new NHHDC) |  |
| 4 | From Participant Id | text(4) | = ID of new NHHDC |  |
| 5 | To Role Code | text(1) | = Z (Non-Core - PARMS) |  |
| 6 | To Participant Id | text(4) | = POOL |  |
| 7 | Creation Time | date/time | Date & time of file generation |  |
| SUB - Subject Participant Header |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = SUB |  |
| 2 | Market Sector | text(1) | = N (indicates NHH data) |  |
| 3 | Market Participant Role Code | text(1) | = D (old NHHDC) |  |
| 4 | Market Participant Id | text(4) | ID of old NHHDC, where unknown enter ‘UUUU’ |  |
| 5 | Period End Date | date | Date of last day of calendar month |  |
| 6 | Periodicity | text(1) |  | ‘M’onthly |
| NC11 Data |  |  |  |  |
| Field | Field Name | Type | Comments |  |
| 1 | Record Type | text(3) | = 1NC |  |
| 2 | GSP Group Id | text(2) | Where unknown ‘_U’ should be used |  |
| 3 | Supplier ID | text(4) | The Market Participant ID of the associated Supplier |  |
| 4 | Count of unique registrations held in previous 14 months | int(7) |  |  |
| 5 | Count of registrations where no D0010 and D0152 held | int(7) |  |  |
| 6 | Count of D0010 and D0152s missing before R1 | int(7) |  |  |
| 7 | Count of D0010 and D0152s missing before R2 | int(7) |  |  |
| 8 | Count of D0010 and D0152s missing before R3 | int(7) |  |  |
| 9 | Count of D0010 and D0152s missing before RF | int(7) |  |  |
| 10 | Count of D0010 and D0152s missing after RF | int(7) |  |  |

Backus-Naur Form:

Missing NHH Meter Reads & History from Old NHHDC to New NHHDC::= ZHD {SUB {1NC}} ZPT

# Appendix 1:	Algorithm For Checksum Required For BSCCo Defined Software

Introduction

In order to meet the Auditor requirements for a method of ensuring file integrity, a checksum algorithm was agreed that provides a signature for files coming into, and out from, the three Business Process Operations (BPO) Service developed systems.

The purpose of the algorithm is to provide a reasonable degree of assurance that the binary nature of the file has not changed.  It does not provide any assurance as to the accuracy of the data within the file.

Requirement

The checksum algorithm is defined as follows:

Wj,k is the kth 4-byte word of the Jth record.

Therefore the function can be defined as:

(((W1,1 XOR W1,2 ) XORW1,3.).. XORW1+n,m)

n = the total number of records (less the footer record)

m = the number of 4 byte words in the last record

Key points to note are that the algorithm:

- excludes line terminators

- excludes the last record

- where the last word of a record is less than 4 bytes then it is padded with binary zeroes.

Pseudo-Code

The detail of the algorithm is as follows (note that this assumes that each record is submitted one by one):

This takes four byte sections (excluding the end of line character), padded with nulls if required, and exclusive OR (XOR) them into checksum. The algorithm for this is illustrated by the following ‘C-like’ pseudo code.

num_chars = strlen (context.record buffer)

FOR (i = 0; i < num_chars;)

value = 0

FOR (j = 0; j < 4; i++, j++)

IF i < num_chars

value = ((value << 8 ) + context.record_buffer[i])

ELSE

value = value << 8

END IF

ENDFOR

context.checksum = context.checksum XOR value

ENDFOR

Example Algorithm

This algorithm has been produced by the BSCCo in Visual Basic. This routine has been used in the generation of test data and the output of the routine has been checked against BPO Service data.

Public Function Calc(sFile As String) As Long

'Algorithm to Calculate Checksum from G. Swinton, 23/04/97.

On Error GoTo c1_Error

Dim i As Integer

Dim j As Integer

Dim chksum As Long

Dim bytes(4) As Integer

Dim sLine As String

' validate that the file exists

Open sFile For Input As #1

Do While Not EOF(1)

Line Input #1, sLine

i = 1

If Left$(sLine, 3) <> "ZPT" Then

For j = 1 To Len(sLine)

bytes(i) = bytes(i) Xor Asc(Mid$(sLine, j, 1))

i = i + 1

If i = 5 Then i = 1

Next

End If

Loop

For j = 1 To 4

chksum = chksum + (256 ^ (4 - j) * bytes(j))

Next

Calc = chksum

c1_Exit:

Close #1

Exit Function

c1_Error:

MsgBox Error$()

Calc = -1

Resume c1_Exit

End Function

Please note: CP1334, CP1344 and CP1348 were implemented as part of the June 2011 Release, effective from 01 July 2011