Balancing and Settlement Code BSC PROCEDURE SHARED SVA METER ARRANGEMENT OF HALF HOURLY IMPORT AND EXPORT ACTIVE ENERGY BSCP550 Version 14.1 Date:

BSC Procedure 550

relating to

# Shared SVA Meter Arrangement of Half Hourly Import and Export Active Energy

1.	Reference is made to the Balancing and Settlement Code (the Code) for the Electricity Industry in Great Britain and, in particular, to the definition of "BSC Procedure".

2.	This is BSC Procedure 550, Version 14.1 relating to Shared SVA Meter Arrangement of Half Hourly Import and Export Active Energy.

3.	This BSC Procedure is effective from.

4.	This BSC Procedure has been approved by the Panel.

Intellectual Property Rights, Copyright and Disclaimer The copyright and other intellectual property rights in this document are vested in Elexon or appear with the consent of the copyright owner. These materials are made available for you for the purposes of your participation in the electricity industry. If you have an interest in the electricity industry, you may view, download, copy, distribute, modify, transmit, publish, sell or create derivative works (in whatever format) from this document or in other cases use for personal academic or other non-commercial purposes. All copyright and other proprietary notices contained in the document must be retained on any copy you make. All other rights of the copyright owner not expressly dealt with above are reserved. No representation, warranty or guarantee is made that the information in this document is accurate or complete. While care is taken in the collection and provision of this information, Elexon Limited shall not be liable for any errors, omissions, misstatements or mistakes in any information or damages resulting from the use of this information or action taken in reliance on it.

# AMENDMENT RECORD

| Version | Date | Description of Changes | Changes Included | Mods/ Panel/ Committee Refs |
| --- | --- | --- | --- | --- |
| 2.0 | Code Effective Date | Approved for use by the Panel |  |  |
| 3.0 | Code Effective Date | Incorporation of SMRS to CMRS Transfers. (Including peer review comments) | NCR114 |  |
| 4.0 | 08/03/02 | P67 ‘Facilitation of Further Consolidation Options for Licence Exempt Generators (DTI Consolidator Working Group ‘Option 4’)’ changes embodied. | AR1177 | SVG12/153 |
| 5.0 | 26/04/02 | MTC changes embodied to identify actual and pseudo MSIDs. | CP765 | SVG14/179 |
| 6.0 | 01/08/03 | Updated for Modification P62 | P62 | SVG29/390 |
| 7.0 | 28/08/03 | August 03 SVA Document Batch Release | CP854 CP912 | SVG29/389 SVG30/397 |
| 8.0 | 04/11/03 | SVA November 03 Release | CP811 | SVG33/442 |
| 9.0 | BETTA Effective Date | SVA February 05 Release and BETTA 6.3 | CP1091 BETTA 6.3 | SVG48/004 |
| 10.0 | 23/08/07 | P197 Release | P197 CP1176 | 115/04 SVG67/16 ISG68/02 |
| 11.0 | 28/02/08 | February 08 Release | CP1214 | SVG81/01 |
| 12.0 | 06/11/08 | November 08 Release | CP1234 | SVG85/02 |
| 13.0 | 28/06/12 | June 12 Release | CP1369 | SVG135/03 |
| 14.0 | 22/02/18 | February 2018 Release | CP1483 | SVG197/04 |
| 14.1 |  | REC Transition |  |  |

# CONTENTS

# Page Number

1.	Introduction	5

1.1	Scope and Purpose of the Procedure	5

1.2	Main Users of Procedure and their Responsibilities	5

1.3	Use of the Procedure	7

1.4	Balancing and Settlement Code Provision	8

1.5	Associated BSC Procedures	9

1.6	Acronyms and Definitions	9

2.	Not Used	9

3.	Interface and Timetable Information	10

3.1	New Connection or Transfers from CMRS to SMRS  	10

3.2	Change of Supplier for an Existing Shared SVA Metering System	13

3.3	Change of HHDC for an Existing Shared SVA Metering System 	26

3.4	Change of Meter Operator Agent for an Existing Shared SVA Metering System	28

3.5	Energise a Shared SVA Metering System 	30

3.6	De-energise a Shared SVA Metering System	32

3.7	Physical Disconnection of a Shared SVA Metering System Following De-energisation	34

3.8	HHDC Collects and Sends Consumption Data	36

3.9	Determination of Measurement Class for Import Site	37

4.	Appendices	38

4.1	Start of Appointment Notification to HHDC	38

4.2	Data Splitting Algorithm	39

4.2.5	Permitted Variations on the Four Standard Methods	43

4.2.5.1	Splitting of Energy in Proportion to Non-Settlement Meter Readings	43

4.2.5.2	Use of Standard Methods with More Than Two Suppliers	46

4.2.5.3	Rounding to One Decimal Place	46

4.3	Rules for Allocation Schedules	46

4.4	Procedure for Splitting the Export or Import Active Energy Data Between Suppliers	48

4.5	Check Supplier Notifications of Registration Details	49

4.6	Validation Rules to be Followed by HHDC	49

4.7	Missing or Invalid Period Data	50

4.8	Termination of Shared SVA Meter Arrangements	50

# 1.	Introduction

## [MEM]1.1	Scope and Purpose of the Procedure

This BSC Procedure (BSCP) defines the additional processes and responsibilities involved where two or more Suppliers receive Active Energy through the same Shared SVA Metering System, referred to as Shared SVA Meter Arrangement.

This data shall be supplied to meet either the:-

(a)	Initial Volume Allocation Run timescales; or

(b)	Final Reconciliation Volume Allocation Run timescales.

The requirements for allocating the Reactive Energy between two or more Suppliers are outside the scope of this document (although nothing in this BSCP shall preclude Suppliers and HHDCs from applying a Shared SVA Metering Arrangement to Reactive Energy as well as Active Energy).

Since Suppliers involved with Shared SVA Meter Arrangement do not have to appoint the same Half Hour Data Aggregator (HHDA), the “Change of HHDA” procedure is not covered in this document but in BSCP503.

This BSCP should also be used in conjunction with BSCP68 for the Transfers of Registrations of Third Party Generator / Generating Plant from CMRS to SMRS. However, in the case of a Transfer from SMRS to CMRS do not use the disconnection process described in this BSCP, but refer directly to BSCP68.

If the site is considered (by the SVA MOA and HHDC) to be a Complex Site, refer to Retail Energy Code (REC) Metering Operations Schedule for details regarding the D0268 `Half Hourly Meter Technical Details’ dataflow and the `Complex Site Supplementary Information Form’.

## [MEM]1.2	Main Users of Procedure and their Responsibilities

This BSCP should be used by Suppliers, acting in the role of a Primary or Secondary Supplier, Supplier Agent(s), (including SVA Meter Operator Agents, Half Hourly Data Collectors, and Half Hourly Data Aggregators), each Licensed Distribution System Operator (LDSO) and the Transfer Co-ordinator.

The Suppliers using the Shared SVA Meter Arrangements shall agree who is to act as the Primary Supplier and who is to act as Secondary Supplier or Secondary Suppliers in the case of the Multiple Fixed Block Method for Shared SVA Meter Arrangements.  Each Supplier will allocate the appropriate half hourly Meter Timeswitch Code (MTC) for each Metering System Identifier (MSID) (either actual Primary MSID or pseudo Secondary MSID(s)) that they are responsible for as defined in Market Domain Data.

In addition for the Fixed Block and Multiple Fixed Blocks Methods, the Primary Supplier will be required to identify which Party is the Variable Supplier and which Parties are the Fixed Supplier(s).

The Primary Supplier is responsible for nominating the SVA Meter Operator Agent (SVA MOA) and the Half Hourly Data Collector (HHDC) Qualified in the procedures set out in this document for the Primary and Secondary Shared SVA MSIDs and for informing the Secondary Supplier or in the case of the Multiple Fixed Block Method informing the Secondary Suppliers of these nominations.

The number of Suppliers that can utilise the Multiple Fixed Block Method, for either the Active Import or the Active Export Energy will be set at eight, however this flexible limit can be increased if all organisations involved can do so without compromising the processes or timescales described in BSCP550.  Where eight Suppliers choose to utilise the Multiple Fixed Block Method, this will result in one Primary Supplier (utilising a Primary MSID) and seven Secondary Suppliers utilising pseudo Secondary MSIDs (with a single Variable Supplier having two MSIDs) sharing the Active Energy through the same Shared SVA Metering System.

Where the same Metering Equipment (ME) is being utilised for the measurement of Import and Export Energy, the Export Supplier (or Primary Export Supplier, as the case may be) shall liaise with the Import Supplier (or Primary Import Supplier, as the case maybe) to obtain the identity of the SVA MOA to be appointed (in accordance with the Retail Energy Code) to comply with the requirements of the Code. Similarly, where a common Outstation is being utilised for the Import and Export Energy the Export Supplier (or the Primary Export Supplier, as the case maybe) shall liaise with the Import Supplier (or Primary Import Supplier, as the case may be) and appoint the same HHDC.  In the event that a Supplier’s HHDC is not Qualified for Shared SVA Meter Arrangement, the Suppliers shall agree to appoint the same Qualified HHDC.

The Primary Supplier is responsible for the Primary MSID and for the associated new pseudo Secondary MSID(s) until the change of Supplier process is completed.  Each Secondary Supplier will take subsequent responsibility for a pseudo Secondary MSID.

Each Supplier is responsible for appointing a SVA MOA, HHDC and HHDA for their respective Shared SVA MSID and for maintaining its own registration details in SMRS.  In respect of the SVA MOA and HHDC appointments, all Suppliers shall appoint the same Party Agents.

The Suppliers shall ensure the Allocation Schedule for splitting the half hourly Active Energy data between them complies with the Code requirements, as set out in the relevant Appendices and is provided to the HHDC by Gate Closure. If the Allocation Schedule is received after Gate Closure for the start of the Settlement Day to which it is related, then that Allocation Schedule will not be used by the HHDC, and the existing one will be used instead.

The LDSO is responsible for maintaining a record of the Primary and pseudo Secondary MSIDs it has issued, together with the association between them (i.e. that there is only one physical Metering System (MS)), and for ensuring no duplicates are created.

The SVA MOA and HHDC shall maintain similar records. The SVA MOA shall record against the Identifier for the relevant Associated Supplier the SVA Metering System Number relating to that Supplier (being, in the case of the Supplier which is not the Primary Supplier, the Secondary SVA Metering System Number for each Secondary SVA Metering System) and the Identifiers for the Associated Half Hourly Data Collector.

Where the paragraph above applies, the SVA MOA shall record the same Meter Technical Details for the relevant Metering System against each of the relevant SVA Metering System Numbers, save that, in respect of the SVA Metering System Number relating to the Associated Supplier which is not the Primary Supplier, the value for the pulse multiplier and the Meter multiplier shall be shown as zero.

At the time of being appointed or de-appointed to a shared metering arrangement by a Supplier (Primary or Secondary) the SVA MOA, HHDC and HHDA, as appropriate, shall use reasonable endeavours to confirm that the appointment details are correct and consistent. Where there is an error in appointment details then these will be resolved with the relevant Supplier.

## [MEM]1.3	Use of the Procedure

This BSCP shall be followed at the stage that it is identified that the Active Energy is to be split between Suppliers or that the Shared SVA Meter Arrangement is to cease.  Up to that stage, the procedures set out in BSCP502 shall be followed. In the case of the SVA MOA this procedure sets out how the standard activities defined in the REC Metering Operations Schedule should be performed (and supplemented) in the context of a Shared SVA Meter Arrangement. Upon the appointment of two or more Suppliers to a new Shared SVA Metering System, or more than one Supplier for an existing SVA Metering System, the Suppliers shall ensure that an Allocation Schedule, as set out in the relevant Appendix, is agreed.  This Allocation Schedule shall be based upon an approved Method for splitting the Active Energy data between the Suppliers.

The HHDC shall receive an initial Allocation Schedule from the Primary Supplier and the Primary Supplier will be the single Party that has the authority of all of the other Parties involved for providing updates to the Allocation Schedule.

The HHDC shall allocate the proportion of half hourly Active Energy data specified in the Allocation Schedule.  There are four Methods:

- Percentage Method;

- Capped Block Method;

- Fixed Block Method; or

- Multiple Fixed Block Method.

Details of each data splitting method are covered in the relevant Appendix.  In addition, Appendix 4.2.5 describes permitted variations on the four standard Methods that Suppliers may use (provided that their HHDC has systems and processes to support them). If the Allocation Schedule is not provided or is invalid, the HHDC will apply the rules described in the relevant Appendix.

The Primary MSID is the enduring MSID for any changes between single and Shared SVA Meter Arrangements and shall have the MTD for the physical MS assigned to it.  The pseudo Secondary MSID(s) shall only exist whilst there is Shared SVA Meter Arrangement.  The MTD for pseudo Secondary MSID(s) shall be amended such that the Meter Multiplier and Pulse Multiplier are set to zero, to minimise the risk of duplicate metering data entering the Settlement process.

The determination of Measurement Class for the Import Site requires special processing by the Suppliers.  Suppliers therefore have a choice of:

Calculating the Measurement Class based on the total consumption at the Site; or

Registering the Site as Measurement Class ‘C’, and thereby treating it as above 100kW for performance assurance purposes.

When the Primary Supplier completes the registration of the pseudo Secondary MSID(s) for a new connection (3.1) or change of Supplier (3.2), the Supply Start Date (SSD) specified shall be sufficiently in advance of the required contract SSD to enable the Secondary Supplier(s) to undertake the change of Supplier process by the contract SSD, taking into account the 10-day lockout period.  The pseudo Secondary MSID(s) shall be registered as de-energised during this interim period to avoid erroneous metering data being passed into Settlements.  Also, for this interim period the Primary Supplier shall appoint a SVA MOA, HHDC and HHDA for this pseudo Secondary MSID(s).

Throughout this procedure, timetables reflect the number of Working Days (WDs) by which an activity should be completed.

The remaining sections in this document are:

Section 2 - This section is no longer in use.

Section 3 - Interface and Timetable Information:  this section defines in detail the requirements of each business process.

Section 4 - Appendices:  this section defines in greater detail the procedures and responsibilities of the HHDC.

## 1.4	Balancing and Settlement Code Provision

This BSCP has been produced in accordance with the provisions of the Balancing and Settlement Code (the Code).  In the event of an inconsistency between the provisions of this BSCP and the Code, the provisions of the Code shall prevail.

## [MEM]1.5	Associated BSC Procedures

| BSCP68 | Transfer of Registration of Metering Systems between CMRS and SMRS. |
| --- | --- |
| BSCP501 | Supplier Meter Registration Service. |
| BSCP502 | Half Hourly Data Collection for Metering Systems Registered in SMRS. |
| BSCP503 | Half Hourly Data Aggregation for Metering Systems Registered in SMRS. |

## 1.6	Acronyms and Definitions

### [MEM]1.6.1	Acronyms

The terms used in this BSC Procedure are defined as follows:

| CMRS | Central Meter Registration Service |
| --- | --- |
| HHDA | Half Hourly Data Aggregator |
| HHDC | Half Hourly Data Collector |
| Id | Identifier |
| LDSO | Licensed Distribution System Operator |
| SVA MOA | Supplier Volume Allocation Meter Operator Agent |
| MS | Metering System |
| MSID | Metering System Identifier |
| MTC | Meter Timeswitch Code |
| MTD | Meter Technical Details |
| SMRA | Supplier Meter Registration Agent |
| SMRS | Supplier Meter Registration Service |
| SSD | Supply Start Date (also known as Effective from Settlement Date {REGI}). |
| SVA | Supplier Volume Allocation |
| WD | Working Day |

### 1.6.2	Definitions

Full definitions of the above acronyms are, where appropriate, included in the Code.

# 2.	Not Used

# 3.	Interface and Timetable Information

## [MEM]3.1	New Connection or Transfers from CMRS to SMRS

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.1.1 | As required. | Agree:- 1. Primary Supplier and Secondary Supplier(s); 2. Fixed and Variable Supplier(s) if either the Fixed Block or Multiple Fixed Block Methods are to be used; and 3. the contract SSD and the initial Allocation Schedule. | Primary Supplier | Secondary Supplier(s) | Allocation Schedule to comply with Appendices 4.3 & 4.4. | By post, fax or electronic media. |
| 3.1.2 | At least 15 WD prior to contract SSD. | Notify LDSO that Shared SVA Meter Arrangement being initiated and that Primary and pseudo Secondary MSIDs will be required. | Primary Supplier | LDSO | Provide details of intended Suppliers and MSIDs required for Shared SVA Meter Arrangement. | By post, fax or electronic media. |
| 3.1.3 | Within 2 WD of 3.1.2. | Allocate Primary and pseudo Secondary MSIDs, record association and ensure no duplication. | LDSO | SMRA | As per BSCP501. | By post, fax or electronic media. |
| 3.1.4 | Within 2 WD of 3.1.2. | Send Primary and pseudo Secondary MSIDs to Primary Supplier. | LDSO | Primary Supplier | As per BSCP501. | By post, fax or electronic media. |
| 3.1.5 | Within 2 WD of 3.1.3. | Record details for Primary and pseudo Secondary MSIDs. | SMRA |  | As per BSCP501. | Internal process. |
| 3.1.6 | At least 11 WD prior to contract SSD. | Complete Primary Supplier and Party Agent registration details for all MSIDs in SMRS, with appropriate SSD. (see Section 1.3) | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.1.7 | Within 2 WD of 3.1.6. | Record registration details for Primary and pseudo Secondary MSIDs in accordance with BSCP501. | SMRA |  | As per BSCP501. | Internal process. |
| 3.1.8 | At least 5 WD before Appointment Date. | Send Party Agent appointment details for the Primary and pseudo Secondary MSIDs. | Primary Supplier | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.6 & 4.7. | Electronic or other agreed method. |
| 3.1.9 | At the same time as 3.1.8 | Notify HHDC and Secondary Supplier(s) of the initial Allocation Schedule. (HHDC and Secondary Supplier(s) should receive the initial Allocation Schedule by Gate Closure) | Primary Supplier | HHDC, Secondary Supplier(s) | Allocation Schedule to comply with Appendices 4.2 & 4.3 | By fax or electronic media. |
| 3.1.10 | Within 2 WD of 3.1.9 | Confirm that Allocation Schedule was received. | HHDC | Primary Supplier, Secondary Supplier(s) |  | Email |
| 3.1.11 | Within 2 WD of 3.1.9. | Advise Secondary Supplier(s) of pseudo Secondary MSID(s), SVA MOA, HHDC and HHDA details. | Primary Supplier | Secondary Supplier(s) |  | By post, fax or electronic media. |
| 3.1.12 | By contract SSD-1WD. | Follow change of Supplier procedure for pseudo Secondary MSID(s). | Secondary Supplier(s) | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.1.13 | Within 2WD of 3.1.11. | Update record for pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process. |
| 3.1.14 | At least 5 WD before Appointment Date. | Send Party Agent appointment details for the pseudo Secondary MSID(s). | Secondary Supplier(s) | SVA MOA, HHDC, HHDA(s) | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.1.15 | Within 2 WD of 3.1.13. | Confirm appointment details are complete and consistent for pseudo Secondary MSID(s). Resolve errors with Secondary Supplier(s). | SVA MOA, HHDC, HHDA(s) | Secondary Supplier(s) | As per BSCP502 /BSCP503 and Retail Energy Code Metering Operations Schedule | Electronic or other agreed method. |
| 3.1.16 | At least 5 WD before Primary Supplier Appointment Termination Date. | Send appointment termination date for pseudo Secondary MSID(s) to SVA MOA, HHDC and HHDA. | Primary Supplier | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendix 4.7. | Electronic or other agreed method. |
| 3.1.17 | Within 2 WD of 3.1.15. | Confirm appointment termination details are complete and consistent. Resolve errors with Primary Supplier. | SVA MOA, HHDC | Primary Supplier | As per BSCP502/Retail Energy Code Metering Operations Schedule and Appendices 4.1 & 4.6. | Electronic or other agreed method.. |
| 3.1.18 | At least 5 WD before Primary Supplier Appointment Termination Date. | Confirm appointment termination details are complete and consistent for pseudo Secondary MSID(s). Resolve errors with Primary Supplier. | HHDA | Primary Supplier | As per BSCP503. | Electronic, or other agreed method |
| 3.1.19 | As soon as practicable after receipt. | Confirm that Allocation Schedule was received by Gate Closure. | HHDC | Primary Supplier, Secondary Supplier(s) |  | Email |

## 3.2	Change of Supplier for an Existing Shared SVA Metering System

### [MEM]3.2.1	Single to Shared SVA Metering System  6

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.2.1.1 | As required. | Agree:- 1 Primary Supplier and Secondary Supplier(s); 2. Fixed and Variable Supplier(s) if either the Fixed Block or Multiple Fixed Block Methods are to be used; and 3. the contract SSD and the initial Allocation Schedule. | Primary Supplier | Secondary Supplier(s) | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By post, fax or electronic media. |
| 3.2.1.2 | Prior to 3.2.1.3 | Notify LDSO that Shared SVA Meter Arrangement being initiated, provide Primary MSID and notify that pseudo Secondary MSID(s) will be required. | Primary Supplier | LDSO | Provide details of intended Suppliers and MSIDs required for Shared SVA Meter Arrangement. |  |
| 3.2.1.3 | At least 15 WD prior to contract SSD. | Follow change of Supplier procedure for Primary MSID with agreed SSD. | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.1.4 | Within 2 WD of 3.2.1.3. | Record details for Primary MSID. | SMRA |  | As per BSCP501. | Internal process |
| 3.2.1.5 | At least 13 WD prior to contract SSD. | Notify change to Primary MSID and apply for the pseudo Secondary MSID(s) and identify as a request for pseudo MSID(s). | Primary Supplier | LDSO | As per BSCP501. | By post, fax or electronic media. |
| 3.2.1.6 | Within 2 WD of 3.2.1.5. | Designate original MSID as Primary MSID, allocate pseudo Secondary MSID(s), record association and ensure no duplication. | LDSO | SMRA | As per BSCP501. | Manual interface. |
| 3.2.1.7 | Within 2 WD of 3.2.1.5. | Send MSIDs to Primary Supplier. | LDSO | Primary Supplier | As per BSCP501. | Electronic or other agreed method. |
| 3.2.1.8 | At least 11 WD prior to contract SSD. | Complete Supplier and Party Agents registration details in for pseudo MSIDs in SMRS, with an appropriate SSD. (see Section 1.3) | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.1.9 | Within 2 WD of 3.2.1.8. | Record details for pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process. |
| 3.2.1.10 | At least 5 WD before Secondary Supplier Appointment Date | Send Party Agent appointment details for the Primary and pseudo Secondary MSID(s). | Primary Supplier | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.2.1.11 | At the same time as 3.2.1.10 | Notify HHDC and Secondary Supplier(s) of the initial Allocation Schedule. (HHDC and Secondary Supplier(s) should receive the initial Allocation Schedule by Gate Closure) | Primary Supplier | HHDC, Secondary Supplier(s) | Allocation Schedule to comply with Appendices 4.2 & 4.3 | By fax or electronic media. |
| 3.2.1.12 | Within 2 WD of 3.2.1.11 | Confirm that Allocation Schedule was received. | HHDC | Primary Supplier, Secondary Supplier(s) |  | Email |
| 3.2.1.13 | Within 2 WD of 3.2.1.11. | Advise Secondary Supplier(s) of pseudo Secondary MSID(s), SVA MOA, HHDC and HHDA details. | Primary Supplier | Secondary Supplier(s) |  | By post, fax or electronic media. |
| 3.2.1.14 | By contract SSD-1WD. | Follow change of Supplier procedure for pseudo Secondary MSID(s). | Secondary Supplier(s) | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.1.15 | Within 2 WD of 3.2.1.13. | Update record for pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process. |
| 3.2.1.16 | At least 5 WD before Secondary Supplier Appointment Date. | Send Party Agent appointment details for the pseudo Secondary MSID(s). | Secondary Supplier(s) | SVA MOA, HHDC. HHDA(s) | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.2.1.17 | At least 5 WD before Primary Supplier Appointment Termination Date. | Send appointment termination date for pseudo Secondary MSID(s) to SVA MOA, HHDC and HHDA. | Primary Supplier | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendix 4.6. | Electronic or other agreed method. |
| 3.2.1.18 | Within 2 WD of 3.2.1.17. | Confirm appointment termination details are complete and consistent. Resolve errors with Primary Supplier. | SVA MOA, HHDC. HHDA | Primary Supplier | As per BSCP502/BSCP503 and Retail Energy Code Metering Operations Schedule. | Electronic or other agreed method. |

### [MEM]3.2.2	Change of All Suppliers for a Shared SVA Metering System6

| REF. |  | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.2.2.1 |  | As required. | Agree:- - Primary Supplier and Secondary Supplier(s); 2. Fixed and Variable Supplier(s) if either the Fixed Block or Multiple Fixed Block Methods are to be used; and 3. Contract SSD and initial Allocation Schedule. | Primary Supplier | Secondary Supplier(s) | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By post, fax or electronic media. |
| 3.2.2.2 |  | Prior to 3.2.2.3 | Notify LDSO of intention to amend Shared SVA Meter Arrangement. | Primary Supplier | LDSO | Provide details of intended Suppliers and MSIDs required for Shared SVA Meter Arrangement. |  |
| 3.2.2.3 |  | At least 1 WD prior to contract SSD. | Follow change of Supplier procedure for Primary MSID with agreed contract SSD. | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.2.4 |  | Within 2 WD of 3.2.2.3 | Record details for Primary MSID. | SMRA |  | As per BSCP501. | Internal process |
| 3.2.2.5 |  | At least 1 WD prior to contract SSD. | Advise Secondary Supplier(s) of SVA MOA, HHDC and HHDA appointment details. | Primary Supplier | Secondary Supplier(s) |  | By post, fax or electronic media. |
| 3.2.2.6 |  | By contract SSD-1WD. | Follow change of Supplier procedure for pseudo Secondary MSID(s), with agreed contract SSD. | Secondary Supplier(s) | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.2.7 |  | Within 2 WD of 3.2.2.6. | Update record for pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process. |
| 3.2.2.8 |  | At least 5 WD before Appointment Date. | Send Party Agent appointment (or de-appointment) details for the Primary MSID. | New Primary Supplier | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.2.2.9 |  | At least 5 WD before Appointment Date. | Send Party Agent appointment (or de-appointment) details for the pseudo Secondary MSID(s). | New Secondary Supplier(s) | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.2.2.10 |  | By Gate Closure. | Notify HHDC and Secondary Supplier(s) of the revised Allocation Schedule. (HHDC and Secondary Supplier(s) shall receive the revised Allocation Schedule by Gate Closure) | Primary Supplier | HHDC, Secondary Suppliers(s) | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By fax or electronic media. |
| 3.2.2.11 | As soon as practicable after receipt. | Confirm that revised Allocation Schedule was received by Gate Closure. | HHDC | Primary Supplier, Secondary Supplier(s) |  | Email. |

### [MEM]3.2.3	Change of Supplier for a Shared SVA Metering System6

This process is applicable for either a change of Primary or Secondary Supplier.  In the case where a Primary Supplier is changing, the reference to Primary Supplier in the table below is to the original Primary Supplier except in 3.2.3.8 and 3.2.3.9 where it will be the new Primary Supplier.

| REF. |  | WHEN | ACTION | FROM |  | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.2.3.1 |  | As required. | Agree contract SSD and revised Allocation Schedule. | Current Supplier(s) |  | New Supplier | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By post, fax or electronic media. |
| 3.2.3.2 |  | Prior to 3.2.3.3 | Notify LDSO of intention to amend Shared SVA Meter Arrangement. |  | Primary Supplier | LDSO | Provide details of intended Suppliers and MSIDs required for Shared SVA Meter Arrangement. |  |
| 3.2.3.3 |  | At least 1 WD prior to contract SSD. | Follow change of Supplier procedure for MSID with agreed contract SSD. | New Supplier |  | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.3.4 |  | Within 2 WD of 3.2.3.3. | Record details for changing Supplier. | SMRA |  |  | As per BSCP501. | Internal process |
| 3.2.3.5 |  | At least 1 WD prior to contract SSD. | Notify new Supplier of SVA MOA, HHDC and HHDA appointment details. | Primary Supplier |  | New Supplier |  | By post, fax or electronic media. |
| 3.2.3.6 |  | At least 5 WD before New Supplier Appointment Date. | Send Party Agent appointment details for the changing MSID. | New Supplier |  | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.2.3.7 |  | At least 5WD before Old Supplier Appointment Termination Date. | Send appointment termination date for changing Supplier to SVA MOA, HHDC and HHDA. | Old Supplier |  | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503 and Retail Energy Code Metering Operations Schedule. | Electronic or other agreed method. |
| 3.2.3.8 |  | By Gate Closure. | Notify HHDC and Secondary Supplier(s) of the revised Allocation Schedule. (HHDC and Secondary Supplier(s) shall receive the revised Allocation Schedule by Gate Closure) | Primary Supplier |  | HHDC, Secondary Suppliers(s) | Allocation Schedule to comply with Appendices 4. 2 & 4.3. | By fax or electronic media. |
| 3.2.3.9 | As soon as practicable after receipt. | Confirm that revised Allocation Schedule was received by Gate Closure. | HHDC |  | Primary Supplier, Secondary Supplier(s) |  | Email. |

### [MEM]3.2.4	Additional Pseudo Secondary MSID(s) to a Shared SVA Metering System with a Multiple Fixed Block Allocation Schedule6

| REF. |  | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.2.4.1 |  | As required. | Agree contract SSD and revised Allocation Schedule. | Primary Supplier | Additional & Current Secondary Suppliers | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By post, fax or electronic media. |
| 3.2.4.2 |  | Prior to 3.2.4.3 | Notify LDSO of intention to amend Shared SVA Meter Arrangement. | Primary Supplier | LDSO | Provide details of intended Suppliers and MSIDs required for Shared SVA Meter Arrangement. |  |
| 3.2.4.3 |  | At least 13 WD prior to contract SSD. | Apply for additional pseudo Secondary MSID(s) and identify as a request for pseudo Secondary MSID(s). | Primary Supplier | LDSO | As per BSCP501. | By post, fax or electronic media. |
| 3.2.4.4 |  | Within 2 WD of 3.2.4.3. | Allocate additional pseudo Secondary MSID(s), record association and ensure no duplication. | LDSO | SMRA | As per BSCP501. | Manual interface. |
| 3.2.4.5 |  | Within 2 WD of 3.2.4.3. | Send MSID details to Primary Supplier for pseudo Secondary MSID(s). | LDSO | Primary Supplier | As per BSCP501. | Electronic or other agreed method. |
| 3.2.4.6 |  | At least 11 WD prior to contract SSD. | Complete Supplier and Party Agents registration details for additional pseudo Secondary MSID(s) in SMRS, with an appropriate SSD. (see Section 1.3) | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.4.7 |  | Within 2 WD of 3.2.4.6. | Record details for additional pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process. |
| 3.2.4.8 |  | At least 5 WD before Primary Supplier Appointment Date. | Send Party Agent appointment details for the additional pseudo Secondary MSID(s). | Primary Supplier | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/ Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.2.4.9 |  | Within 2 WD of 3.2.4.8. | Confirm details for additional MSID are complete and consistent. Resolve errors with Primary Supplier. | SVA MOA, HHDC, HHDA(s) | Primary Supplier | As per BSCP502/BSCP503 and Retail Energy Code Metering Operations Schedule. | Electronic or other agreed method. |
| 3.2.4.10 |  | Within 2 WD of 3.2.4.9. | Advise additional Secondary Supplier(s) of MSID(s), SVA MOA, HHDC and HHDA details. | Primary Supplier | Additional Secondary Supplier(s) |  | By post, fax or electronic media. |
| 3.2.4.11 |  | By contract SSD-1WD. | Follow change of Supplier procedure for new pseudo Secondary MSID(s). | Additional Secondary Supplier(s) | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.4.12 |  | Within 2 WD of 3.2.4.11. | Update record for new pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process. |
| 3.2.4.13 |  | At least 5 WD before Appointment Date. | Send Party Agent appointment details for the new pseudo Secondary MSID(s). | Additional Secondary Supplier | SVA MOA, HHDC. HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.2.4.14 |  | At least 5 WD before Primary Supplier Appointment Termination Date. | Send appointment termination date for additional pseudo Secondary MSID(s) to SVA MOA, HHDC and HHDA. | Primary Supplier | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503 and Retail Energy Code Metering Operations Schedule | Electronic or other agreed method. |
| 3.2.4.15 |  | By Gate Closure. | Notify HHDC and all Secondary Suppliers of the revised Allocation Schedule. (HHDC and Secondary Suppliers shall receive the revised Allocation Schedule by Gate Closure) | Primary Supplier | HHDC, Secondary Suppliers | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By fax or electronic media. |
| 3.2.4.16 | As soon as practicable after receipt. | Confirm that revised Allocation Schedule was received by Gate Closure. | HHDC | Primary Supplier, Secondary Suppliers |  | Email. |

### [MEM]3.2.5	Removal of a Secondary Supplier(s) to a Shared SVA Metering System with a Multiple Fixed Block Allocation Schedule

| REF. |  | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.2.5.1 |  | As required. | Inform Primary Supplier of SSD for removal of pseudo Secondary MSID(s). | De-registering Secondary Supplier(s) | Primary Supplier | MSID to be removed and SSD | By post, fax or electronic media. |
| 3.2.5.2 |  | Prior to 3.2.5.3 | Notify LDSO of intention to amend Shared SVA Meter Arrangement. | Primary Supplier | LDSO | Provide details of intended Suppliers and MSIDs required for Shared SVA Meter Arrangement. |  |
| 3.2.5.3 |  | As required. | Agree revised Allocation Schedule with remaining Secondary Supplier(s) and inform them of the applicable SSD. | Primary Supplier | Remaining Secondary Suppliers | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By post, fax or electronic media |
| 3.2.5.4 |  | At least 1 WD prior to contract SSD. | Follow change of Supplier procedure for MSID(s) with agreed contract SSD. | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.5.5 |  | Within 2 WD of 3.2.5.4. | Update record for de-registering pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process |
| 3.2.5.6 |  | Within 3 WD of 3.2.5.5. | Request logical disconnection with date for de-registering pseudo Secondary MSID(s). | Primary Supplier | LDSO | As per BSCP501. | Electronic or other agreed method. |
| 3.2.5.7 |  | Within 1 WD of 3.2.5.6. | Record de-registering pseudo Secondary MSID(s) as disconnected. | LDSO | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.5.8 |  | At least 1 WD prior to contract SSD. | Confirm disconnection of de-registering pseudo Secondary MSID(s) to Primary Supplier. | LDSO | Primary Supplier | As per BSCP501. | Electronic or other agreed method. |
| 3.2.5.9 |  | At least 5 WD before Secondary Supplier Appointment Termination Date. | Send appointment termination date for de-registering pseudo Secondary MSID(s) to SVA MOA, HHDC and HHDA. | Primary Supplier | SVA MOA, HHDC, HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendix 4.6. | Electronic or other agreed method. |
| 3.2.5.10 |  | Before Secondary Supplier Appointment Termination Date. | Confirm appointment termination date for de-registering MSID with de-registering Supplier. | Primary Supplier | De-registering Supplier | MSID appointment termination date | Electronic or other agreed method |
| 3.2.5.11 |  | By Gate Closure. | Notify HHDC and all existing Secondary Suppliers of the revised Allocation Schedule. (HHDC and Secondary Suppliers shall receive the revised Allocation Schedule by Gate Closure) | Primary Supplier | HHDC, Secondary Suppliers | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By post, fax or electronic media. |
| 3.2.5.12 | Within 1 WD of 3.2.5.11. | Confirm that revised Allocation Schedule was received by Gate Closure. | HHDC | Primary Supplier, Secondary Suppliers |  | Email. |  |

### [MEM]3.2.6	Shared to Single SVA Metering System

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.2.6.1 | Following agreement with other Secondary Suppliers to terminate Shared SVA Meter Arrangement. | Notify LDSO of intention to terminate Shared SVA Meter Arrangement. | Primary Supplier | LDSO |  | Electronic or other agreed method. |
| 3.2.6.2 | At least 5 WD prior to contract SSD. | Follow change of Supplier procedure for Primary and pseudo Secondary MSIDs. | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.6.3 | Within 2 WD of 3.2.6.1. | Update record for Primary and pseudo Secondary MSIDs. | SMRA |  | As per BSCP501. | Internal process |
| 3.2.6.4 | At least 3 WD prior to contract SSD. | Request disconnection with date for pseudo Secondary MSID(s). | Primary Supplier | LDSO | As per BSCP501. | Electronic or other agreed method. |
| 3.2.6.5 | At least 1 WD prior to contract SSD. | Record pseudo Secondary MSID(s) as disconnected and change Primary MSID to normal MSID in records. | LDSO | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.2.6.6 | Within 2 WD of 3.2.6.5. | Update record for Primary and pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process. |
| 3.2.6.7 | At least 1 WD prior to contract SSD. | Confirm disconnection of pseudo Secondary MSID(s) to Supplier. | LDSO | Primary Supplier | As per BSCP501. | Electronic or other agreed method. |
| 3.2.6.8 | At least 5 WD before Appointment Date. | Send appointment details for the remaining MSID to SVA MOA, HHDC and HHDA. | Primary Supplier | SVA MOA, HHDC HHDA | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.2.6.9 | At least 5 WD before Appointment Termination Date. | Send appointment termination date to SVA MOA, HHDC & HHDA(s). | Primary & Secondary Suppliers | SVA MOA HHDC, HHDA(s) | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendix 4.6. | Electronic or other agreed method. |

## [MEM]3.3	Change of HHDC for an Existing Shared SVA Metering System 6

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.3.1 | As required. | Notify SMRA of new HHDC appointment for Primary MSID. | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.3.2 | Within 2 WD of 3.3.1. | Update record for Primary MSID. | SMRA |  | As per BSCP501. | Internal process |
| 3.3.3 | Within 2 WD of 3.3.1. | Notify Secondary Supplier(s) of the new HHDC appointment. | Primary Supplier | Secondary Supplier(s) | As per Appendix 4.8 | By post, fax or electronic media. |
| 3.3.4 | Within 2 WD of 3.3.3. | Update SMRS with new HHDC for pseudo Secondary MSID(s). | Secondary Supplier(s) | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.3.5 | Within 2 WD of 3.3.4. | Update record for pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process |
| 3.3.6 | At least 5 WD before Appointment Date. | Send appointment details of the new HHDC and the termination date for the old HHDC for the Primary MSID. | Primary Supplier | SVA MOA, New HHDC, HHDA, Old HHDC | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.3.7 | At the same time as 3.3.6 | Notify HHDC and Secondary Supplier(s) of the revised Allocation Schedule. (HHDC and Secondary Supplier(s) shall receive the revised Allocation Schedule by Gate Closure) | Primary Supplier | New HHDC, Secondary Supplier(s) | Allocation Schedule to comply with Appendices 4.2 & 4.3 | By fax or electronic media. |
| 3.3.8 | Within 2 WD of 3.3.7 | Confirm that revised Allocation Schedule was received. | HHDC | Primary Supplier, Secondary Supplier(s) |  | Email. |
| 3.3.9 | At least 5 WD before Appointment Date. | Send appointment details of the new HHDC and the termination date for the old HHDC for the pseudo Secondary MSID(s). | Secondary Supplier(s) | SVA MOA, New HHDC, HHDA(s), Old HHDC | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6. | Electronic or other agreed method. |
| 3.3.10 | As required. | Send Meter history details. | Old HHDC | New HHDC | As per BSCP502 |  |

## [MEM]3.4	Change of Meter Operator Agent for an Existing Shared SVA Metering System6

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.4.1 | As required. | Notify SMRA of new SVA MOA appointment for Primary MSID. | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.4.2 | Within 2 WD of 3.4.1. | Update record for Primary MSID. | SMRA |  | As per BSCP501. | Internal Process |
| 3.4.3 | Within 2 WD of 3.4.1. | Notify Secondary Supplier(s) of new SVA MOA appointment. | Primary Supplier | Secondary Supplier(s) | As per Appendix 4.8 | By post, fax or electronic media. |
| 3.4.4 | Within 2 WD of 3.4.3. | Update SMRS with new SVA MOA details for pseudo Secondary MSID(s). | Secondary Supplier(s) | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.4.5 | Within 2 WD of 3.4.4. | Update record for pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal Process |
| 3.4.6 | At least 5 WD before Appointment Date. | Send appointment details of the new SVA MOA and termination of old SVA MOA for the Primary MSID. | Primary Supplier | New SVA MOA, Old SVA MOA, HHDC | As per BSCP502/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6 | Electronic or other agreed method. |
| 3.4.7 | At least 5 WD before Appointment Date. | Send appointment details of the new SVA MOA and termination date for the Old SVA MOA for the pseudo Secondary MSID(s). | Secondary Supplier(s) | New SVA MOA, Old SVA MOA, HHDC | As per BSCP502/Retail Energy Code Metering Operations Schedule and Appendices 4.1, 4.5 & 4.6 | Electronic or other agreed method. |
| 3.4.8 | At least 5 WD before Appointment Termination Date. | Send appointment termination date for Primary MSID to old SVA MOA. | Primary Supplier | Old SVA MOA | As per Retail Energy Code Metering Operations Schedule | Electronic or other agreed method. |
| 3.4.9 | At least 5 WD before Appointment Termination Date. | Send appointment termination date for pseudo Secondary MSID(s) to old SVA MOA. | Secondary Supplier(s) | Old SVA MOA | As per Retail Energy Code Metering Operations Schedule | Electronic or other agreed method. |
| 3.4.10 | Before Appointment Termination Date. | Send MTD. | Old SVA MOA | New SVA MOA | As per Retail Energy Code Metering Operations Schedule. |  |

## [MEM]3.5	Energise a Shared SVA Metering System 6

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.5.1 | As required. | Agree:- 1. date supply is to be energised; 2. the initial Allocation Schedule; and 3. whether LDSO or SVA MOA is to energise supply. | Primary Supplier | Secondary Supplier(s) | Allocation Schedule to comply with Appendices 4.2 & 4.3. | By post, fax or electronic media. |
| 3.5.2 | At least 2 WD prior to planned energisation date. | Notify SMRA of planned energisation date for Primary MSID. | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.5.3 | Within 2 WD of 3.5.2. | Update record for Primary MSID. | SMRA |  | As per BSCP501. | Internal Process |
| 3.5.4 | At least 2 WD prior to planned energisation date. | Notify SMRA of date of planned energisation date for Secondary pseudo MSID(s). | Secondary Supplier(s) | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.5.5 | Within 2 WD of 3.5.4. | Update record for Secondary pseudo MSID(s). | SMRA |  | As per BSCP501. | Internal Process |
| 3.5.6 | At least 5 WD prior to energisation Date. | Send date for planned energisation for the Primary MSID to the LDSO or SVA MOA and HHDC. | Primary Supplier | LDSO or SVA MOA/HHDC | As per BSCP502 and Retail Energy Code Metering Operations Schedule. | Electronic or other agreed method. |
| 3.5.7 | At least 5 WD prior to energisation Date. | Send date for planned energisation for the Secondary pseudo MSID(s) to the LDSO or SVA MOA and HHDC. | Secondary Supplier(s) | LDSO or SVA MOA/HHDC | As per BSCP502 and Retail Energy Code Metering Operations Schedule. | Electronic or other agreed method. |
| 3.5.8 | On energisation Day. | Carry out energisation and obtain reading for physical MS. Liaise with HHDC to collect HH data | SVA MOA or LDSO | HHDC. |  | Electronic or other agreed method. |
| 3.5.9 | Within 2 WD of 3.5.8. | Send date of energisation for the MSID to Primary and Secondary Suppliers and HHDC. Confirm that: a) energisation is complete including energisation dates and (for a physical energisation) MS reading; and b) details for all MSIDs are complete and consistent. Resolve errors with Suppliers and notify SVA MOA or LDSO. | LDSO or SVA MOA | Primary & Secondary Suppliers, HHDC | As per BSCP502 and Retail Energy Code Metering Operations Schedule. | Electronic or other agreed method. |
| 3.5.10 | Following energisation date. | Notify of actual energisation date (which may be different from planned energisation date). | Primary and Secondary Suppliers | SMRA, HHDC | As per BSCP501 | Electronic or other agreed method. |
| 3.5.11 | Upon receiving actual Energisation date. | Update SMRA with actual energisation date. | SMRA |  | As per BSCP501 | Internal Process |
| 3.5.12 | Within 2 WD of 3.5.10. | Confirm energisation details are complete and consistent. Resolve errors with Suppliers. | HHDC | Primary & Secondary Suppliers | As per BSCP502/Retail Energy Code Metering Operations Schedule and Appendices 4.4 & 4.6. | Electronic or other agreed method. |
| 3.5.13 | By Gate Closure. | Notify HHDC and Secondary Supplier(s) of the revised Allocation Schedule. (HHDC and Secondary Supplier(s) shall receive the revised Allocation Schedule by Gate Closure) | Primary Supplier | HHDC, Secondary Supplier(s) | Allocation Schedule to comply with Appendices 4.2 & 4.3 | By fax or electronic media. |
| 3.5.14 | As soon as practicable after receipt. | Confirm that revised Allocation Schedule was received by Gate Closure. | HHDC | Primary Supplier, Secondary Supplier(s) |  | Email. |

## [MEM]3.6	De-energise a Shared SVA Metering System

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.6.1 | As required. | Agree:- 1. date supply is to be de-energised; and 2. whether LDSO or SVA MOA is to de-energise supply. | Primary Supplier | Secondary Supplier(s) | As per BSCP502 and Retail Energy Code Metering Operations Schedule. | By post, fax or electronic media. |
| 3.6.2 | At least 2 WD prior to planned de-energisation date. | Notify SMRA of planned de-energisation date for Primary MSID. | Primary Supplier | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.6.3 | Within 2 WD of 3.6.2. | Update record for Primary MSID. | SMRA |  | As per BSCP501. | Internal process |
| 3.6.4 | At least 2 WD prior to planned de-energisation date. | Notify SMRA of planned de-energisation date for pseudo Secondary MSID(s). | Secondary Supplier(s) | SMRA | As per BSCP501. | Electronic or other agreed method. |
| 3.6.5 | Within 2 WD of 3.6.4. | Update record for pseudo Secondary MSID(s). | SMRA |  | As per BSCP501. | Internal process. |
| 3.6.6 | At least 5 WD prior to de-energisation Date. | Send date for de-energisation for the Primary MSID to the HHDC and LDSO or SVA MOA. | Primary Supplier | HHDC & LDSO or SVA MOA | As per BSCP502/Retail Energy Code Metering Operations Schedule and Appendices 4.5 & 4.6. | Electronic or other agreed method. |
| 3.6.7 | At least 5 WD prior to planned de-energisation Date. | Send date for planned de-energisation for the pseudo Secondary MSID(s) to the HHDC and LDSO or SVA MOA. | Secondary Supplier(s) | HHDC & LDSO or SVA MOA | As per BSCP502/Retail Energy Code Metering Operations Schedule and Appendices 4.5 & 4.6. | Electronic or other agreed method. |
| 3.6.8 | On de-energisation Day. | Carry out de-energisation and obtain reading for physical MS. Liaise with HHDC to collect HH data. | SVA MOA or LDSO | HHDC |  | Electronic or other agreed method. |
| 3.6.9 | Within 2 WD of 3.6.8. | Send date of de-energisation for the MSID. Confirm that: - de-energisation is complete including date and MS reading; and - details for all MSIDs are complete and consistent. Resolve errors with Suppliers and notify LDSO or SVA MOA. | LDSO or SVA MOA | Primary and Secondary Suppliers, HHDC | As per BSCP502/Retail Energy Code Metering Operations Schedule and Appendices 4.5 & 4.6. | Electronic or other agreed method. |
| 3.6.10 | Following de-energisation date. | Notify of actual de-energisation date (which may be different from planned de-energisation date). | Primary and Secondary Suppliers. | SMRA, HHDC |  | Electronic or other agreed method. |
| 3.6.11 | Upon receiving actual De-energisation date. | Update SMRA with actual de-energisation date. | SMRA |  | As per BSCP501 | Internal process |
| 3.6.12 | Within 2 WD of 3.6.6. | Confirm that details for all MSIDs are complete and consistent. Resolve errors with Suppliers. | HHDC | Suppliers | As per BSCP502/Retail Energy Code Metering Operations Schedule and Appendix 4.5. | Electronic, or other agreed method. |

## [MEM]3.7	Physical Disconnection of a Shared SVA Metering System Following De-energisation

| REF. | WHEN | ACTION | FROM | TO |  | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.7.1 | As required. | Agree date that Shared SVA Metering System is to be disconnected. | Primary Supplier | Secondary Supplier(s) |  |  | By post, fax or electronic media. |
| 3.7.2 | At least 5 WD prior to disconnection Date. | Send date for the Primary MSID to be disconnected. | Primary Supplier | LDSO |  | As per BSCP502. | By post, fax or electronic media. |
| 3.7.3 | At least 5 WD prior to disconnection Date. | Send date for the Secondary pseudo MSID(s) to be disconnected. | Secondary Supplier(s) | LDSO |  | As per BSCP502. | By post, fax or electronic media. |
| 3.7.4 | By physical disconnection date. | Arrange for physical Shared SVA Metering System associated with the Primary and pseudo Secondary MSIDs to be disconnected, provided details are complete and consistent. Resolve errors with LDSO. | LDSO | SMRA, Suppliers |  | As per BSCP501. | Electronic, or other agreed method. |
| 3.7.5 | Within 1 WD of 3.7.4. | Record intended disconnection date for Primary and pseudo Secondary MSIDs. | SMRA |  |  | As per BSCP501. | Internal process. |
| 3.7.6 | Within 5 WDs of actual disconnection | Update/confirm disconnection date for MSIDs. | LDSO | SMRA |  |  | By post, fax or electronic media. |
| 3.7.7 | At least 5 WD before Appointment Termination Date. | Send appointment termination date to SVA MOA, HHDC & HHDA for Primary MSID. | Primary Supplier | SVA MOA HHDC HHDA(s) |  | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.5 & 4.6. | Electronic or other agreed method. |
| 3.7.8 | Within 2 WD of 3.7.6. | Confirm details are complete and consistent. Resolve errors with Supplier. | SVA MOA, HHDC, HHDA(s) | Primary Supplier | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendix 4.6. |  | Electronic or other agreed method. |
| 3.7.9 | At least 5 WD before Appointment Termination Date. | Send appointment termination date to SVA MOA, HHDC & HHDA(s) for pseudo Secondary MSID(s). | Secondary Supplier(s) | SVA MOA, HHDC, HHDA(s) | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendices 4.5 & 4.6. |  | Electronic or other agreed method. |
| 3.7.10 | Within 2 WD of 3.7.8. | Confirm details are complete and consistent. Resolve errors with Suppliers. | SVA MOA, HHDC, HHDA(s) | Secondary Supplier(s) | As per BSCP502/BSCP503/Retail Energy Code Metering Operations Schedule and Appendix 4.6. |  | Electronic or other agreed method. |

## 3.8	HHDC Collects and Sends Consumption Data

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.8.1 | As agreed with Suppliers’ schedule. | Collect and validate the metering data for the physical Shared SVA Metering System as per BSCP502. The Shared SVA Metering System data is split between the Primary MSID and the pseudo Secondary MSID(s) in accordance with the method specified in the Allocation Schedule | HHDC |  | As per BSCP502 and Appendix 4.4. Utilise the Allocation Schedule. | Internal process. |
| 3.8.2 | As agreed with Suppliers’ schedule. | Send validated metered data to relevant Suppliers, and HHDA and LDSO. | HHDC | Primary & Secondary HHDAs, Primary & Secondary Suppliers and LDSO | Appendix 4.4. D0036 Validated Half Hourly Advances for Inclusion in Aggregated Supplier Matrix. D0275 Validated Half Hourly Advances. | Electronic or other agreed method. |
| 3.8.3 | As agreed with LDSO. | Send validated raw metered data to LDSO. | HHDC | LDSO | Validated raw meter data (based on D0010 Meter Reading and / or D0036 Validated Half Hourly Advances for Inclusion in Aggregated Supplier Matrix, as agreed between the HHDC and the LDSO). | Email |

## 3.9	Determination of Measurement Class for Import Site

| REF. | WHEN | ACTION | FROM | TO | INFORMATION REQUIRED | METHOD |
| --- | --- | --- | --- | --- | --- | --- |
| 3.9.1 | As agreed with Suppliers’ schedule. | Primary and Secondary Suppliers agree Measurement Class based on the total consumption at the Site or alternatively select Measurement Class C for the Site. | Primary Supplier. | Secondary Supplier(s). |  | Manual Process. |
| 3.9.2 | As agreed with Suppliers’ schedule. | Send Measurement Class details. | Primary Supplier. | HHDC, SMRA. | D0289 Notification of MC/EAC/PC. D0055 Registration of Supplier to Specified Metering Point or D0205 Update Registration Details as appropriate. | Electronic or other agreed method. |
| 3.9.3 | As agreed with Suppliers’ schedule. | Send Measurement Class details. | Secondary Supplier(s). | SMRA. | D0055 Registration of Supplier to Specified Metering Point or D0205 Update Registration Details as appropriate. | Electronic or other agreed method. |

# 4.	Appendices

## [MEM]4.1	Start of Appointment Notification to HHDC

The HHDC will initially receive a notification of start of appointment from the Primary Supplier (for the Primary MSID and each pseudo Secondary MSID).  Subsequently the HHDC will receive a change of appointment from each Secondary Supplier (for each associated pseudo secondary MSID).

The HHDC can only be appointed by one Primary Supplier at any one time and may additionally be appointed to one or more Secondary Suppliers at any one time.

The HHDC should check that the notification is received at least 5 WDs in advance of the appointment date to allow sufficient time for the HHDC to co-ordinate the resolution of any control issues which may arise.

The HHDC shall then check that the following information is included in the notification, in addition to the standard information included in notifications relating to the single Supplier arrangements:

1.	the identity of the intended Secondary Supplier(s); and

2.	the Allocation Schedule to be used.

In relation to the appointment of the HHDC to the Secondary Supplier(s), in addition to the standard information included in notifications the following information will be provided:

3.	notifications relating to the Primary and pseudo Secondary MSID(s) have been received, and there are more than 5 WDs before the HHDC appointment start date;

4.	the Primary MSID is notified as appointed to the Primary Supplier and the pseudo Secondary MSID(s) is/are notified as appointed to the Secondary Supplier(s);

5.	Allocation Schedule has been received correctly and conform to the requirements in the relevant Appendix;

6.	there is one and only one SVA MOA and HHDC notified as appointed to all MSIDs for a Shared SVA Meter Arrangement; and

7.	unless agreement of all the organisations involved has been provided, that no more than eight pseudo Secondary MSIDs participate in the Shared SVA Meter Arrangement.

If a HHDC receives notification of start of appointment to one or many pseudo Secondary MSIDs, but not to a Primary MSID, the HHDC should report the error to the appointing Supplier or Suppliers and BSCCo.  If the HHDC does not receive notification of start of appointment from a Secondary Suppler, the HHDC will report this to the Primary Supplier.

## 4.2	Data Splitting Algorithm

# This Appendix 4.2 describes four standard Methods for splitting of half hourly Active Energy. Suppliers using these standard Methods are able, via the Primary Supplier, to nominate to the HHDC use of one and (subject to Appendix 4.2.5) only one of the four Methods for a given Settlement Period:

- Percentage Method;

- Capped Block Method;

- Fixed Block Method; or

- Multiple Fixed Block Method.

# The agreed Method will be provided to the HHDC by the Primary Supplier in the Allocation Schedule.  Any Allocation Schedule received from any other source will be rejected by the HHDC.

# Subject to Appendix 4.2.5, a maximum of two Suppliers can establish a Shared SVA Meter Arrangement for Percentage, Capped Block and Fixed Block Methods, whereas more than two Suppliers can establish a Shared SVA Meter Arrangement for a Multiple Fixed Block Method.  Each of the Methods are detailed below.

# Appendix 4.2.5 describes permitted variations on the four standard Methods that Suppliers may use (provided that their HHDC has systems and processes to support them).  All such permitted variations remain subject to the requirements on Allocation Schedules and splitting of data in Appendices 4.3 and 4.4.

### 4.2.1	Percentage Method

# This Method allows two Suppliers, a Primary and Secondary Supplier to split Active Energy on a percentage basis.  Percentage Method allocations are supplied by the Primary Supplier for both the Primary and pseudo Secondary MSIDs.

# The Primary Supplier nominates an % amount and the Secondary Supplier is allocated the remainder.

# Example:

| Settlement Period |  | Active Energy | Primary Supplier Active Energy | Secondary Supplier Active Energy |
| --- | --- | --- | --- | --- |
| 13 | % | 100 | 70 | 30 |
|  | kWh | 30 | 21 | 9 |
| 14 | % | 100 | 60 | 40 |
|  | kWh | 20 | 12 | 8 |
| 15 | % | 100 | 80 | 20 |
|  | kWh | 40 | 32 | 8 |

# Where the Percentage Method is used, the potential exists for non-integer values of kWh to be allocated. Rounding rules will be applied to prevent non-integer values of energy being processed by Settlements. Where, after the HHDC has applied the percentage split to derive the Primary Supplier’s allocation:

- the value has a non-integer component greater than 0.5 kWh, the value will be rounded up to the next integer value e.g. 4.6 kWh will be rounded up to 5 kWh;

- the value has a non-integer component less than 0.5 kWh, the value will be rounded down to the next integer value e.g. 4.4 kWh will be rounded down to 4 kWh;

- the value has a non-integer component equal to 0.5 kWh, the value will be rounded up to the next integer value when the Settlement Period is an odd number e.g. 4.5 kWh will be rounded up to 5 kWh in Settlement Periods 1, 3, 5 etc.; and

- the value has a non-integer component equal to 0.5 kWh, the value will be rounded down to the next integer value when the Settlement Period is an even number e.g. 4.5 kWh will be rounded down to 4 kWh in Settlement Periods 2, 4, 6 etc.

# The value allocated to the Primary Supplier will be subtracted from the actual metered quantity and the balance allocated to the Secondary Supplier.

### 4.2.2	Capped Block Method

# This Method allows two Suppliers, a Primary and Secondary Supplier to split Active Energy on a capped block basis.  The Primary Supplier nominates an integer amount of kWh and the Secondary Supplier is allocated the balance.  In circumstances where the Primary Supplier nominates an amount greater than the actual Active Import or Export Energy in a given Settlement Period zero kWh will be allocated to the Secondary Supplier and the Primary Supplier will be allocated the actual Active Import or Export Energy for that Settlement Period.

# Example:

| Settlement Period | Active Energy kWh | Primary Supplier Nominated kWh | Primary Supplier Actual Active Energy kWh | Secondary Supplier Actual Active Energy kWh |
| --- | --- | --- | --- | --- |
| 13 | 100 | 60 | 60 | 40 |
| 14 | 20 | 30 | 20 | 0 |
| 15 | 40 | 40 | 40 | 0 |
| 16 | 0 | 40 | 0 | 0 |

### [HK]4.2.3	Fixed Block Method

# This Method allows two Suppliers, a Primary and Secondary Supplier to split Active Energy with one Supplier (Fixed Supplier) nominating a fixed integer amount of kWh and the other Supplier (Variable Supplier) being allocated the variable amount.  Either the Primary or Secondary Supplier can be nominated as taking the fixed or variable blocks.  If Suppliers wish to change roles as either a Variable or Fixed Supplier a minimum of 5 WDs notice must be given to the HHDC.

# The value allocated to the Fixed Supplier will be subtracted from the actual metered quantity and the balance allocated to the Variable Supplier.

# Where the fixed amount of Active Import is greater than the actual Active Import Energy the balance will be recorded as virtual Active export Energy against the Variable Supplier. For the purposes of Settlement, the Consumption Component Class for the virtual Active export Energy will be recorded as Active Export Energy.

# Where the fixed amount of Active Import is less than the actual Active Import Energy the balance will be recorded as an Active Import Energy against the Variable Supplier.

# Where the fixed amount of Active Export is greater than the actual Active Export Energy the balance will be recorded as virtual Active Import Energy against the Variable Supplier. For the purposes of Settlement, the Consumption Component Class for the virtual Active import Energy will be recorded as Active Import Energy.

# Where the fixed amount of Active Export is less than the actual Active Export Energy the balance will be recorded as a Active Export Energy against the Variable Supplier.

# The Fixed Block Method Allocation Schedule will be deemed to be invalid if Import or Export of Active Energy allocated to the Fixed Supplier is greater than the Relevant Capacity.

# Example:

# In this example the Active Energy is Export, the same principle is applicable to Import.

| Settlement Period | Actual Active Export Energy (kWh) | Fixed Supplier Nominated Active Export (kWh) | Fixed Supplier Active Export (kWh) | Variable Supplier |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Active Export (kWh) | Virtual Active import (kWh) |
| 12 | 100 | 100 | 100 | 0 | 0 |
| 13 | 100 | 60 | 60 | 40 | 0 |
| 14 | 50 | 40 | 40 | 10 | 0 |
| 15 | 20 | 30 | 30 | 0 | 10 |
| 16 | 0 | 60 | 60 | 0 | 60 |

### [HK]4.2.4	Multiple Fixed Block Method

This Method is an expansion of the Fixed Block Method and allows more than two Suppliers, a Primary and many Secondary Suppliers, to split Active Energy.  The Active Energy is split into fixed integer blocks of kWhs to be taken by Fixed Suppliers and the variable element of energy is allocated to one Supplier, the Variable Supplier.  If Suppliers wish to change roles as either a Variable or Fixed Supplier a minimum of 5 WDs notice must be given to the HHDC.  However, in all applications of the Multiple Fixed Block Method there shall be only one Supplier nominated as the Variable Supplier.

Any Supplier in the Multiple Fixed Block Method can be nominated as the Primary Supplier and all the other Suppliers will be treated as Secondary Suppliers.

The value allocated to the Fixed Suppliers will be subtracted from the actual metered quantity and the balance allocated to the Variable Supplier.

Where the sum of the fixed amounts of Active Import is greater than the actual Active Import Energy the balance will be recorded as virtual Active Export Energy against the Variable Supplier. For the purposes of Consumption Component Class, virtual Active export Energy will be recorded as Active Export Energy.

Where the sum of the fixed amount of Active Import is less than the actual Active Import Energy the balance will be recorded as an Active Import Energy against the Variable Supplier.

Where the sum of the fixed amount of Active Export is greater than the actual Active Export Energy the balance will be recorded as virtual Active import Energy against the Variable Supplier. For the purposes of Consumption Component Class, virtual Active import Energy will be recorded as Active Import Energy.

Where the sum of the fixed amount of Active Export is less than the actual Active Export Energy the balance will be recorded as an Active Export Energy against the Variable Supplier.

The Multiple Fixed Block Method Allocation Schedule will be deemed to be invalid if Import or Export of Active Energy allocated to the Fixed Suppliers is greater than the Relevant Capacity20.

Example:

In this example the Active Energy is Export and the group of Fixed Suppliers are identified as Fixed Supplier A, B, C, D and E.

| Settlement Period | Active Energy kWh Export | Fixed Suppliers Active Export (kWh) |  |  |  |  | Variable Supplier 21 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | Active Export (kWh) | Virtual Active import (kWh) |
| 13 | 100 | 10 | 5 | 20 | 15 | 10 | 40 | 0 |
| 14 | 50 | 5 | 10 | 15 | 10 | 10 | 0 | 0 |
| 15 | 20 | 2 | 8 | 5 | 5 | 10 | 0 | 10 |
| 16 | 0 | 2 | 8 | 5 | 5 | 10 | 0 | 30 |

## 4.2.5	Permitted Variations on the Four Standard Methods

# Where the relevant Suppliers agree (and provided that their appointed HHDC has systems and processes to support it) the four standard Methods for splitting of half hourly Active Energy may be varied as follows.

## 4.2.5.1	Splitting of Energy in Proportion to Non-Settlement Meter Readings

# Suppliers may establish a Shared SVA Metering Arrangement in which Active Import and/or Active Export Meter readings (recorded at the Settlement Boundary Point) are apportioned between Suppliers based on readings from non-Settlement Meters on a private network.  This allows generators and/or customers connected to the private network to contract with third party Suppliers, while ensuring an appropriate treatment of losses on the private network.

# In this context, the Allocation Schedule will identify:

- The non-Settlement Meter(s) associated with each Supplier.  Note that these Meters must be capable of providing half hourly data, and must be recording the same Measurement Quantity as the Settlement Meter at the Boundary Point.

- The agreed method for allocating between Suppliers the ‘unaccounted for’ Active Energy (i.e. the difference between the Boundary Point Meter reading and the total of the non-Settlement Meter readings).  Such ‘unaccounted for’ Active Energy may be allocated in proportion to the non-Settlement Meter readings, or using other methods agreed by the Suppliers.

# Allocating Unaccounted For Energy in Proportion to Non-Settlement Meter Readings

# Where the Allocation Schedule specifies that ‘unaccounted for’ Active Energy is to be allocated in proportion to the non-Settlement Meter readings, the HHDC will calculate the Meter reading Mj for each Supplier i as:

# Meter Reading for Supplier i = Msett x Mi / ∑ Mi

# where Msett is the Half Hourly Meter reading at the Settlement boundary, and Mi is the reading from each Supplier’s non-Settlement Meter.

# Figure 1 provides an example of the type of site for which Suppliers may wish to use this arrangement.  Two generators are connected to a private network, each with their own Export Meter (M1 and M2) which is Settlement-standard (i.e. compliant with the relevant Code of Practice) but not a Settlement Meter (as it is not directly associated with any Settlement Metering System).  There are two Metering Systems registered for the site, but these are Shared SVA Metering Systems, each associated with the single Settlement Meter M3.  The HHDC will use the Export readings from the non-Settlement Meters M1 and M2 to determine the allocation of the Settlement Export Meter reading M3 between the two Suppliers:

# Active Export Meter Reading for Supplier 1 = M3 x M1 / (M1 + M2)

# Active Export Meter Reading for Supplier 2 = M3 x M2 / (M1 + M2)

# This ensures that all of the electrical losses on the private network (and any metering errors in the non-Settlement Meters) are allocated between the two Suppliers at the site.

# Other Methods of Allocating Unaccounted For Energy

# Other methods of allocating the unaccounted for Active Energy are permissible, provided that they are agreed by all the Suppliers; that they are supported by the Qualified HHDC’s systems and processes; and that they are consistent with the requirements in Appendices 4.3 and 4.4 (e.g. the requirement to agree the Allocation Schedule and notify it to the HHDC prior to Gate Closure).

# For example, consider a private network on which one or more customers (with non-Settlement Meters) have contracted with third party Suppliers, while the remainder buy their power from the landlord.  The Suppliers involved have agreed that the HHDC should allocate the Active Import recorded on the Boundary Meter in each Settlement Period as follows:

- Each of the third party Suppliers shall be allocated the Active Import recorded on their customer’s non-Settlement Meter, adjusted by an agreed factor to allow for losses on the private network; and

- The landlord’s Supplier shall be allocated the remaining Active Import.

# In this case the Allocation Schedule would identify the agreed loss adjustment factor Fi for each Supplier i.  The HHDC would calculate the Meter Reading for each Supplier i (other than the landlord’s Supplier) as follows:

# Meter Reading for Supplier i = Mi x Fi

# and the Meter Reading for the landlord’s Supplier as:

# Meter Reading for Landlord’s Supplier = Msett – ∑ (Mi x Fi)

# where Msett is the Half Hourly Meter reading at the Settlement boundary, and Mi is the reading from each Supplier’s non-Settlement Meter.

# Treatment of Metering Faults in Non-Settlement Meters

# Regardless of the method used to allocate ‘unaccounted for’ Active Energy, the Suppliers must agree (in advance of Gate Closure, and as part of the Allocation Schedule if appropriate) how Active Energy should be apportioned in the event that Meter readings from one or more of the non-Settlement Meters are unavailable.  For example, the Allocation Schedule could include a default allocation to be used in the event of non-Settlement metering faults.

# Use in Combination with Standard Methods

# Where the relevant Suppliers agree (and provided that their appointed HHDC has systems and processes to support it) the splitting of Active Energy in proportion to non-Settlement Meter readings may be combined with other standard Methods.  For example, the splitting based on non-Settlement Meters could be applied first, to split the Meter reading at the boundary between two (or more) virtual Metering Systems.  A second round of splitting (using one of the standard Methods) would then be applied to each virtual Metering System.

## 4.2.5.2	Use of Standard Methods with More Than Two Suppliers

# Where the relevant Suppliers agree (and provided that their appointed HHDC has systems and processes to support it) standard Methods (such as the Percentage and Fixed Block methods) may be extended to more than two Suppliers.

## 4.2.5.3	Rounding to One Decimal Place

# Where the relevant Suppliers agree (and provided that their appointed HHDC has systems and processes to support it) Meter readings calculated using the standard Methods may be rounded up or down to a tenth of a kWh (rather than up or down to a whole number of kWh, as required by the standard Percentage Method described in Appendix 4.2.1).  The rounding must still be carried out in such a way as to ensure that the Active Energy allocated to the Suppliers sums (without rounding errors) to the Active Energy recorded on the Settlement Metering System.

## 4.3	Rules for Allocation Schedules

# 4.3.1	Allocation Schedules for a Whole Settlement Day

The HHDC is required to use Allocation Schedules provided by the Primary Supplier to split the Active Import or Export Energy.

The HHDC will validate the Allocation Schedule using the following rules:

1.	the Primary Supplier is the Party who  has the authority of all the Parties involved to routinely provide Allocation Schedule details;

2.	The initial Allocation Schedule shall endure until replaced by a revised valid Allocation Schedule.  If the revised Allocation Schedule, following validation, is invalid, then the initial Allocation Schedule will be used by the HHDC until a replacement valid Allocation Schedule is received except as described in Section 4.3.2;

3.	each Allocation Schedule must apply to one or more Settlement Day, except in the special case as described in Section 4.3.2;

4.	the Allocation Schedule must specify which data splitting method is to be used, i.e. Percentage Method, Capped Block Method, Fixed Block Method, Multiple Fixed Block Method or a permitted variation on the standard Methods as described in Appendix 4.2.5;

5.	Allocation Schedules must be uniquely identifiable for each Settlement Day, differentiating between versions where used;

6.	If the Fixed Block or the Multiple Fixed Block Method is to be used, that the maximum output or consumption for the Plant or Apparatus is specified;

7.	That no negative quantity is specified; and

8.	If a Capped Block Method, Fixed Block Method or Multiple Fixed Block Method is specified, the blocks specified are integer values of kWhs of zero or greater.

The Allocation Schedules must apply to each Settlement Period in a Settlement Day.

Daily re-nomination of parameters in the Allocation Schedule is allowed but they must be finalised prior to Gate Closure for the required Settlement Day. Allocation Schedules may not be altered for Settlement Periods for which Gate Closure has passed.  A log of such exceptions and reasons must be maintained.

In the event that an initial Allocation Schedule is not received by Gate Closure, the HHDC shall allocate 100% of the consumption for each Settlement Period for the appropriate Settlement Days to the Primary Supplier.

If the subsequent Allocation Schedule that relates to a whole Settlement Day is received from the Primary Supplier after Gate Closure, for the start of the Settlement Day to which it is related, then for Settlement Periods for which Gate Closure has passed the subsequent Allocation Schedule will not be used, and the existing one will be rolled forward and used for the Settlement Period(s) for which Gate Closure has already passed.  Each Allocation Schedule must include the following details:

1.	Primary MSID and associated Primary Supplier;

2.	Pseudo Secondary MSID(s) and associated Secondary Supplier(s);

3.	allocation type (Percentage, Capped Block, Fixed Block, Multiple Fixed Block Method or a permitted variation on the standard Methods as described in Appendix 4.2.5);

4.	Settlement Day(s); and

5.	the Settlement Period volume parameters, set as a percentage of output, specified in whole percentage points, or a block(s) of kWh.

The HHDC shall maintain a copy of each valid Allocation Schedule that it receives in its system.

#### 4.3.2	Allocation Schedules for Part of a Settlement Day

If the Primary Supplier chooses to send revised Allocation Schedules relating to part of a Settlement Day then the HHDC will accept the revised Allocation Schedule (subject to the other validation rules in 4.3.1) providing Gate Closure has not been reached for the relevant Settlement Period (and subject to the constraints specified in 4.2.3 and 4.2.4).

## 4.4	Procedure for Splitting the Export or Import Active Energy Data Between Suppliers

All Allocation Schedules received from the Primary Supplier will be recorded.

The HHDC must process the Active Export Energy or the Active Import Energy meter reading data collected and allocate the output between the Suppliers using one of the Methods as outlined in the relevant Appendix and the authorised Allocation Schedule. In the absence of half hourly Active Energy data due to Metering Equipment faults, etc., or where the data fails the normal validation criteria (see BSCP502), the HHDC shall estimate the meter data in accordance with the validation rules found in BSCP502. The data validation processes shall be carried out on the original metered data collected for the physical MS prior to any splitting.

The HHDC is required to:

1.	use the authorised Allocation Schedules received as the basis for the splitting process;

2.	allocate for each Settlement Period the split Export or Import Active Energy meter data to the Primary and pseudo Secondary MSIDs;

3.	ensure that in each Settlement Period the sum of the outputs (taking into account the Measurement Quantity) allocated to each Supplier is equal to the data recorded by the MS;

4.	send each Supplier, and its appointed HHDA, the split meter data allocated to its MSID; and

5.	send all Supplier MSID data allocations to the LDSO.

The HHDC must ensure that the output data splitting process is verifiable.  This means that:

1.	the original values input to the data splitting Method and the calculated output values are retained; and

2.	an audit trail is maintained between the Export or Import meter data, the Allocation Schedule used, the data allocated to each Supplier MSID and the data sent to the LDSO, HHDAs and Suppliers.

## [MEM]4.5	Check Supplier Notifications of Registration Details

The HHDC is required to check the completeness, consistency and continuity of details of SMRS registration notifications provided to the HHDC by the Suppliers(s).

The following checks need to be carried out:

1.	Registration notifications need to be received, within the required time period, for all MSIDs involved in data split instructions;

2.	Registration notifications for each Supplier need to be consistent with each other, consistent with the HHDC’s own historical records of previous notifications and consistent with the MTD received from the SVA MOA; and

3.	For the events of new connection, disconnection, energisation and de-energisation of the MTD, registration notifications for Primary and pseudo Secondary MSID(s) must be consistent.

Error reports should be sent to the appropriate Supplier(s) if discrepancies are found.

The HHDC is required to maintain audit records of the checks carried out and the results, including details of any errors reported to Suppliers.

## [MEM]4.6	Validation Rules to be Followed by HHDC

The validation to be undertaken by an HHDC shall be as follows:

1.	an appointment shall be received from Suppliers relating to their respective MSIDs which refers to the same Physical Metering System as defined by the Meter Serial Number (or numbers where the Physical Metering System comprises more than one Meter);

2.	one appointment shall state that the Supplier is the Primary Supplier and the other shall state that the other Supplier(s) are/is the Secondary Supplier(s);

3.	all appointments shall name the same SVA Meter Operator Agent;

4.	all appointments shall be for the same date at least 5 WDs ahead.  Where the date is not 5 WDs ahead the HHDC shall request the Supplier to  set the appointment dates to 5 WDs ahead of the latest appointment;

5.	the initial Allocation Schedule and associated maintenance rules shall be received from the Primary Supplier;

6.	the Primary Supplier is the Party which is deemed to have  the authority of all involved Parties to routinely provide Allocation Schedule details;

7.	the MTD received from the SVA Meter Operator Agent for each MSID refers to the same physical Metering System as defined by the meter serial number(s); and

8.	the Meter and Pulse Multipliers for the pseudo Secondary MSID(s) are set to zero.

## 4.7	Missing or Invalid Period Data

The HHDC shall estimate the total half hourly metered data values, for any Shared SVA Meter Arrangement, in accordance with BSCP502.  The HHDC shall record and utilise for estimation purposes those values provided by the Associated Primary Supplier or the SVAA, in accordance with BSCP502.

The HHDC shall provide an Estimated Data Report to the Associated Primary Supplier and LDSO. This report shall identify all SVA MSs for which meter period data has been estimated showing the dates and Settlement Periods affected and details of the estimation methods used.

The estimated total half hourly metered data value will be the value to which the valid Allocation Schedule will be applied.

## 4.8	Termination of Shared SVA Meter Arrangements

The HHDC shall cease to undertake its duties and functions in accordance with this BSCP:

1.	where it is notified by the Associated Primary Supplier that the Shared SVA Meter Arrangement is terminated and once there is only a single Supplier appointed to the relevant SVA Metering Equipment.

Where this appendix has effect, the HHDC shall, as soon as is reasonably practicable, notify its Associated Suppliers and the relevant LDSO.

The Approved Headline Report for SVG136 contains the SVG’s decision to move CP1369 to the June 12 Release

This section numbering used in the BSC Procedure must not change in order to keep the document in line with the other BSCPs listed in Section 1.5.

[MEM]

[MEM]

If a Registration Transfer from CMRS, the Supply Start Date will be the “Effective From Date” defined in BSCP68, Section 3.2.

Each Supplier will allocate the appropriate half hourly MTC for each MSID (either actual Primary MSID or pseudo Secondary MSID(s)) that they are responsible for as defined in Market Domain Data.

[MEM] Refer to Retail Energy Code Metering Operations Schedule for the method of installation of the MS.

If Registration is a Transfer of Registration from CMRS, proceed in accordance with BSCP68.

Where the existing Supplier continues on as either the Primary or Secondary Supplier, the appropriate parts of this procedure shall be followed.

Where either of the existing Suppliers continue on as the sole Supplier, only the appropriate parts of this section of this BSCP shall be followed.

If a HHDC does not receive notification of a start of appointment by the Primary Supplier, although he may have received a start of appointment from the Secondary Supplier(s) refer to Appendix 4.1.

Where a common Outstation is being utilised for the Import and Export Energy, the Primary Supplier shall ensure that there are no technical issues where different HHDCs collect from the same Outstation.

[MEM]

For a pseudo MSID, energisation will be a logical not a physical process.

For a pseudo MSID, de-energisation will be a logical not a physical process.

For a pseudo MSID, disconnection will be a logical not a physical process.

The HHDC shall record the total half hourly metered data values collected from the relevant SVA Metering Equipment in a separate file marked to identify the relevant SVA Metering Equipment and shall maintain records so as to identify the Metering Equipment with the relevant SVA MS Numbers.

On each occasion that the HHDC applies a valid Allocation Schedule in respect of the total half hourly metered data values from the relevant SVA Metering Equipment to provide an output file of data to the HHDA, it shall record a history identifying the output file reference, the Allocation Schedule and the metered data for the relevant day to which it was applied.

The same Measurement Class must be agreed for use at both the Primary and Secondary MSIDs.

Where Relevant Capacity is defined at Section K 2.2.5 (g) of the Code.

Where the Variable Supplier is nominated as exporting or importing for a Settlement Period, then the HHDC will need to ensure that a value is present for either the export or the import MSID but not for both MSIDs. Where the Active Energy equals the Fixed Supplier’s nominated value, a value of zero must be present for both the export and the import MSIDs.